| Firmware Version: | 1.01r006 |
| --- | --- |
| Published Date: | 2024/04/23 |

Content:

Upgrading Instructions:	2

Upgrading by using Web-UI	2

New Features:	2

Changes of MIB & D-View Module:	2

Problems Fixed:	3

Known Issues:	3

Related Documentation:	3

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |

Upgrading Instructions:

D-Link AP supports firmware upgrading via Web-UI. You may download the firmware from D-Link web site http://tsd.dlink.com.tw.

Please also

- Make sure your DAP-X3060 has to manually assign a valid static IPv4 address or get a valid IPv4 address from DHCP server before upgrade FW.

- Do not power off your DAP-X3060 when FW upgrading.

Upgrading by using Web-UI

- Connect a work station to the LAN port of Access Point.

- Open the web browser of work station and entering the IP address of the AP. The system default IP address setting is dynamic ip, and you can enter http://dapx3060.local to login, if it cannot get ip address from DHCP server, it’s ip address will back to 192.168.0.50.

- Enter administrator’s username and password, user name and password are “admin” by default.

To update the AP's firmware, click Maintenance > Firmware and SSL Certification Upload

New Features:

- Enable DFS channel for FCC & IC

- Support click through feature in standalone moe

Changes of MIB & D-View Module:

None

Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.01 | - [DBG] Fix Bad Gateway issue - [D-Track]Fix WMM for multi SSID1~7 can be selected Enable or Disable |

Known Issues:

| Firmware Version | Known Issues |
| --- | --- |
| V1.01 | None |

Related Documentation:

DAP-X3060 user manual v1.00