DBT-122

Date: March 14, 2005

| Product name: | USB Bluetooth Adapter |  |
| --- | --- | --- |
| Revision: | A1 |  |
| Available: | M/ Feb. 2005 |  |
| Picture: |  |  |
| Why Sell This Product: | - We’re releasing this new Bluetooth adapter to supplement our current DBT-120 Bluetooth adapter. The new DBT-122 adapter complies with the newer 1.2 Bluetooth standard, supports the Adaptive Frequency Hopping (AFH) function that resolves all channel utilization conflicts with any Wi-Fi equipment operating in the same area, and is targeted at the main Bluetooth markets such as Europe. The current DBT-120 will continue to sell in the U.S., where it has been popular. - The new DBT-122 will eventually replace the DBT-120 in the future. Selling Points - Works with Windows-based PCs, including Win2000 and XP. - Supports Adaptive Frequency Hopping (AFH) function that automatically selects to use channels not utilized by nearby operating Wi-Fi equipment. - Supports more Bluetooth profiles. |  |
| Key Features: | - USB connection to PC - Up to 723Kbps asymmetrical, 433.9Kbps symmetrical data transfer rate - Bluetooth SIG 1.2 compliant - Supports Adaptive Frequency Hopping (AFH) function - RF wireless frequency 2400to 2483.5 MHz for USA, Europe, Japan, 2446.5 to 2483.5 MHz for France - 128-bit data encryption - 2dBi on-board antenna - Up to 10 meters (33 ft.) wireless operation range* * Environmental factors may adversely affect operation range. |  |
| Technical Details: | System Requirements - PC equipped with 300MHz processor or above - 64 MB RAM or more - USB 1.1 or 2.0 port - Microsoft Windows XP, 2000, ME or 98SE with USB 1.1/2.0 support - CD-ROM drive (for device driver installation) Bluetooth Standard Bluetooth SIG 1.2 Host Interface USB 1.1 Bluetooth Profile - DUN: DT/GW - Fax: DT/GW - FTP: client/server - HCRP: client - Headset: AG/HS - HID: host - OPP: client/server - PAN: AP/group/user - SPP: Dev A/Dev B - Sync: client /server - LAN: DT/GW - Generic Access - Basic Image RF Emission Type Frequency Hopping Spread Spectrum (FHSS) RF Modulation Type GFSK RF Frequency - 2400 to 2483.5MHz for USA, Europe, Japan - 2446.5 to 2483.5 MHz for France Transmit Output Power -6 to +4dBm (Class 2) Receive Sensitivity -83 dBm typical across pass band under high interference environments Antenna 2dBi on-board General Function Supports up to 7 slave devices Transfer Rates - Up to 723 Kbps asymmetrical - Up to 433.9 Kbps symmetrical Data Encryption 128-bit Wireless Operation Range Up to 10 meters (33 feet) peer-to-peer * * Environmental factors may adversely affect operation range. Physical & Environmental Power Supply USB powered (derived from PC) Dimensions 47 x 19 x 10 mm Weight 78 grams Operating Temperature 0 to 50 ℃ Storage Temperature -20 to 65 ℃ Operating Humidity 5% to 95% non-condensing Emission Certification - FCC Class B - CE - C-Tick - VCCI - BSMI O.S. Driver Support Windows XP, 2000, ME, 98 SE Package Includes - DBT-122 USB adapter - USB extension cable - CD (containing User's Manual) - Quick Installation Guide |  |
| EMI/EMC Report | FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others |  |
| Safety |  TUV No.  UL No.  CSA No.  Others |  |
| Packaging Style |  Clamshell  White Box  Gift Box  Bulk  White Box with D-Link Logo  Others |  |
| Packaging Contents: | - DBT-122 Device - USB Extension Cable - CD-ROM with Mannual - QIG |  |
| UPC Code: | BBT122….A1; 790069 276040. IBT122….A1; 790069 276057. IBT122SG..A1; 790069 276064. EBT122….A1; 790069 276071. NBT122….A1; 790069 276088. TBT122….A1; 790069 276095. |
| Units Per Carton: | 40 |  |
| Cartons Per Pallet: | For USA Air: , Sea: | For WW Air:12, Sea: 20 |
| Dimensions and Weight: | Box: ,Others , 0.27 kg Carton: 495 x 410 x 360 mm, 11.7 kg Pallet: (by Air) mm, kg (for USA) 1200 X 1100 X1230 mm, 162 kg (for WW) (by Sea) mm, kg (for USA) 1200 x 1100 x 1950 mm, 255 kg (for WW) INTRASTAT weight: 0.126 kg (Box) Packing material weight (Box): - Paper: 0.098 kg - Plastic: 0.044 kg - Mixed: 0.004 kg |  |