Pls. Translate below English and keep original English words.

##### Page1

This product works with Bluetooth enabled PCs, Mobile Phones, and PDAs

PersonalAir™ Bluetooth USB Printer Adapter

Before You Begin

You must have at least the following:

•	Bluetooth Enabled PC Supporting HCRP and SPP Bluetooth 			Profiles for Printing, Bluetooth Enabled Mobile Phone, or Bluetooth 		enabled PDA

•	USB Printer* with One Available Type B USB Port

(*PCL3 Printer is required for Printing from Cell Phone or PDA)

Check Your Package Contents

These are the items included with your purchase:

•	DBT-320 Bluetooth USB Printer Adapter

• Quick Installation Guide

• Wall Mount Cradle

• 5V 2.0A DC Power Adapter

If any of the above items are missing, please contact your reseller.

©2004 D-Link Systems,Inc. All rights reserved. Trademarks or registered trademarks are the property of their respective holders. Software and specifications subject to change without notice. DBT-320. 02272004

###### Page2

Installing the DBT-320

Plug the DBT-320 Type B USB connector into the Type B USB port located on the back of your printer.

Using the included power adapter, connect the yellow coded plug to the yellow power connector port on the DBT-320 and the other end of the power connector to an available power strip or outlet.

The DBT-320 Bluetooth USB Printer Adapter set up is now complete! You will now be able to enjoy wireless printing from your cell phone, PDA, laptop, or computer with the DBT-320 Bluetooth USB Printer Adapter.

Page3

DBT-320 LED Indicators

Power Indicator

Bluetooth Link Indicator

The DBT-320 features two LED indicators, the Ready LED and the BT LED.

•	The Ready LED indicator will light a solid green when the DBT-320 is powered on with the external power adapter. When the printer is powered on, a blinking green Ready LED indicates a successful connection with the printer.

•	The BT Indicator will blink green when a file is being sent to the printer from a laptop, computer, PDA, or cell phone.

Printing from your Laptop/PC using the DBT-320

Your Bluetooth enabled laptop or computer will have Bluetooth software that will enable you to connect to the DBT-320 printer adapter.  The instructions below are based on a D-Link DBT-120 USB Bluetooth adapter used with a PC running Windows XP with Widcomm Bluetooth Software v1.4.2.10.  Depending on your Bluetooth setup on your laptop or computer, your installation may vary slightly from the steps below.

Page4

Printing from your Laptop/PC using the DBT-320 (continued)

Double-click the Bluetooth icon on your desktop or in the systray to access My Bluetooth Places.

You can also access My Bluetooth Places from the Start Menu:  Go to Start > Programs > My Bluetooth Places

Click Search for devices in range

printer icon will be displayed*.  If the DBT-320 is not detected, press F5 on your keyboard or right click using your mouse and select refresh to search again.

Double Click to install printer drivers

*Note: If the printer is being used by another Bluetooth user, you will not be able to detect the DBT-320.

Page5

Click OK

Select the Manufacturer of your printer and your printer model.

Click Have Disk

Click Browse to search for your printer drivers.

Click Browse

Highlight the printer you are using in the Printers list.

Click OK

Page6

The printer properties window will be displayed once your printer is installed. You can click on Print Test Page to test your connection to the DBT-320.

Click Print Test Page

Printing from your Mobile Phone

To print from your mobile phone, be sure that  you have at least the following:

•	Bluetooth enabled mobile phone supporting the Bluetooth 	Object Push Profile which will enable you to print business cards 		from your mobile phone

•	PCL3 printer

The Bluetooth printing procedure will vary depending on the manufacturer of your mobile phone.  Please check your mobile phone manual or with your manufacturer for specific directions on how to print.

Page7

Printing from your PDA

To print from your PDA, be sure that you have at least the following:

•	Bluetooth enabled PDA

•	Bluetooth Software Supporting the  Object Push Profile

•	Printing software (i.e PrintPocket CE for PDAs running Pocket PC 	2002 or 2000, BTPrint for Palms running Palm OS 4.x or OS 5.x, 		HP Mobile Printing for iPAQs running Pocket PC 2003)

•	PCL3 printer

The Bluetooth printing procedure will vary depending on the manufacturer of your PDA and printing software that you are using.  Please check your PDA  manual or with your manufacturer for specific directions on how to print.

Troubleshooting Tips

If you are having trouble printing with the DBT-320 check the following:

•	Make sure your printer is turned on and the Ready LED on the

DBT-320 is green. If the Ready LED is not green, check your USB	connection to the printer and your power adapter connection.

•	Make sure the Bluetooth icon on the screen of your laptop, computer,  mobile phone, or PDA is white. This means that  the Bluetooth application is ready. If the Bluetooth icon is not white, try rebooting your Bluetooth mobile device and/or reinstalling your Bluetooth software. Check that your Bluetooth software supports the HCRP, SPP, and/or OPP Bluetooth profiles for printing.

•	If the Bluetooth icon is red, the DBT-320 is not properly connected. 	Check the DBT-320 connection to your printer and your printer 		connection.

•	When printing, the Bluetooth icon will blink green. If the Bluetooth 	LED is not flashing on, check your DBT-320 connection to the 		printer. Your printer may also not be configured correctly.  Check 		your printer configuration.