DBT-900AP

Date: March 23, 2004

| Product name: | Bluetooth-to-LAN Access Point |
| --- | --- |
| Product Group & Type: |  |
| Revision: | A1 |
| Available: |  |
| Picture: |  |
| Why Sell This Product: | - Emerging demand for Bluetooth connection, especially in Europe. - Emerging demand for Bluetooth connection to the Internet. - Boost sale of Bluetooth adapters: you can also sell more DBT-120 and DCF-650BT while selling this DBT-900AP. - More complete Bluetooth product line for you to offer. - Plug and play product, no need for aftersale technical support. Selling Points - PAN Profile support, the latest Bluetooth technology. |
| Keyfeatures: | - Bluetooth specification v1.1 (PAN Profile) compliant - Ethernet connection with DHCP assignment - 10/100BASE-TX Ethernet LAN port with auto-sensing - 10/100Mbps speeds, auto MDI/MDIX plug-and-play connection - Plug-and-play installation, no driver required - LED indicators for Power, Bluetooth Connection and LAN Connectivity status - Security function for user access authorization/authentication - Simultaneous access to Internet/Intranet for up to 7 Bluetooth users - Operation range up to 20 meters (Class 2) - Works seamlessly with desktop/laptop computers and PDAs * * Computers, PDAs and other devices connecting to the DBT-900AP access point must be Bluetooth-enabled with PAN Profile support. |
| Technical Details: | Emission Type Frequency Hopping Spread Spectrum (FHSS) RF Frequency 2400 to 2483.5MHz (Europe & USA, except France) Output Power -6 to +4dBm (Class 2) Modulation Type Gaussian Frequency Shift Keying (GFSK) Frequency Hopping 1600hops/sec. (625us/hop) Receiving Sensitivity -80dBm (Class 2) BER 10E-3 Antenna 2dBi Gain dipole antenna LED Indicators - Power - BT (Bluetooth) - LAN Connectors - Power jack - RJ-45 LAN jack Configuration Reset Reset button Physical & Environmental Power Input Through external power adapter Power Adapter - Auto-switching 100 to 240 VAC, 50/60Hz - 5VDC, 2A Dimensions 104 (L) x 63 (W) x 29 (H) mm (device only) Weight 100 grams (device only) Operation Temperature 5 to 50 C Storage Temperature -25 to 50 C Humidity 5% to 95% non-condensing Emission - FCC Class B - CE Class B Safety LVD (EN60950) |
| EMI/EMC Report | FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others |
| Safety |  TUV No.  UL No.  CSA No.  Others |
| Packaging Style |  Clamshell  White Box  Gift Box  Bulk  Others |
| Packaging Contents: | - DBT-900AP x1, - Quick Installation Guide x1 - Cat. 5 UTP cable x1 - Switch Power Adapter x1 - Mounting cradle x1 |
| UPC Code: | DBT900APA.A1; 790069 26313. DBT900APB.A1; 790069 26314. DBT900APE.A1; 790069 26315. DBT900APN.A1; 790069 26316. |
| Units Per Carton: | 20 |
| Cartons Per Pallet: | Air: 20, Sea: 28 |
| Dimensions and Weight: | Box: 224 x 147 x 77 mm, 0.615 kg Carton: 598 x 384 x 245 mm, 15 kg Pallet: (by Air) 1200 x 1100 x 1570 mm, 315 kg (by Sea) 1200 x 1100 x 1900 mm, 435 kg INTRASTAT weight: kg (Box) Packing material weight (Box): - Paper: kg - Plastic: kg - Mixed: kg |