Bluetooth Modem

Quick Installation Guide

# CONTENTS

# Chapter 1 Introduction	4

# Chapter 2 Hardware Installation	4

# Chapter 3 Start using the DFM-562BT	5

# Chapter 1 Introduction

## This Bluetooth Modem allows you connect your bluetooth-enabled computers to the Internet and use fax service wirelessly.

## Package Contents

The box includes the following items:

- One 56K Bluetooth Modem

- Power Adapter

- One Phone Cable

- One “Quick Installation Guide”

# Chapter 2 Hardware Installation

- Straight up the antenna of the DFM-562BT.

(You may need to adjust the direction of the antenna to get better signal strength.)

- Plug the supplied phone cable into the jack on the back of the DFM-562BT marked “LINE”.

- Plug the other end of the phone cable into the wall socket.

- Plug the power cord of the supplied power adapter into the jack on the back of the DFM-562BT marked “POWER”.

- Plug the power adapter into the power socket.

- Push the power switch to turn on the power of the DFM-562BT.  And the “Power” LED Indicator on the front of the DFM-562BT will be light then.

Note: Push the power switch again to turn off the DFM-562BT.

Once properly installed, the DFM-562BT Bluetooth Modem will be able to be connected to other Bluetooth wireless adapters.  And your connected Personal Air networks should look similar to this:

Computers with DBT-120

# Chapter 3 Start Using DFM-562BT

This modem does not need to install any specific driver; it requires the Bluetooth adapter (for example: D-Link DBT-120 USB Bluetooth Adapter) to be installed on the PC or laptop computer in order to access to the DFM-562BT via Bluetooth.

The step-by-step instructions that follow are shown with Widcom Bluetooth Software that included with D-Link DBT-120 USB Bluetooth Adapter under Windows 98SE/ME/2000.

- Right-click to the “B” logo on the system tray and choose “Explorer” or double click “My Bluetooth Places” to open Bluetooth Adapter.

- Double-Click the “Find Bluetooth Devices” to have USB Bluetooth Adapter to find the DFM-562BT.

Note: Before searching, please make sure the DFM-562BT is in the range, up to 10 meter, from DBT-120 D-Link USB Bluetooth Adapter.

- Once the DFM-562BT is found by USB Bluetooth Adapter, double-click the icon of “DFM-562BT”.

- Right-click the “Dial-up Networking” and choose “Connect Dial-up Networking”.

- A “Connection Wizard” will be shown for you to setup your own Internet connection account.  Please refer to your ISP to setup this Connection Wizard and click “Connect” then.

- Once the “Connected” icon is shown on the system tray,  you’re ready to surf Internet from now!