# Definition of AP Statistics

Winson 2003/9/30

| Item | Definition | Algorithm or Variable |
| --- | --- | --- |
| Transmit Success Rate | The rate of every packets AP transmitted without any error | 100*WIRELESS_STAT_GOOD_TRANSMIT/(WIRELESS_STAT_GOOD_TRANSMIT + WIRELESS_STAT_TX_ERROR) |
| Transmit Retry Rate | The rate of every packets AP retries to transmit | 100*WIRELESS_STAT_TX_TOTAL_RETRIES/WIRELESS_STAT_GOOD_TRANSMIT |
| Receive Success Rate | The rate of every packets AP received without any error | = 100*WIRELESS_STAT_GOOD_RECEIVE /(WIRELESS_STAT_RX_RX_ERROR + WIRELESS_STAT_GOOD_RECEIVE + HW_ERROR) HW_ERROR = WIRELESS_STAT_RX_FCS_ERROR + WIRELESS_STAT_RX_RTS_ERROR + WIRELESS_STAT_ACK_RECEIVE_ERROR + WIRELESS_STAT_RX_RTS_OK |
| Receive Duplicate Rate | The rate of same packets AP received more then once | 100* WIRELESS_STAT_RX_DUPLICATES /WIRELESS_STAT_GOOD_RECEIVE |
| RTS Success Count | The count of successful RTS sent. (A CTS is received after sending RTS) | WIRELESS_STAT_RX_RTS_OK |
| RTS Failure Count | The count of RTS fails to be sent. (No CTS are received after sending RTS). | WIRELESS_STAT_RX_RTS_ERROR |
| RTS Success Rate | The rate of RTS AP sent successfully. (CTS are received after sending RTS) | 100* WIRELESS_STAT_RX_RTS_OK /(WIRELESS_STAT_RX_RTS_OK + WIRELESS_STAT_RX_RTS_ERROR) |
| Transmitted Fragment Count | The count of packets AP transmitted without errors | WIRELESS_STAT_GOOD_TRANSMIT |
| Multicast Transmitted Frame Count | The count of multicast packets AP transmitted successfully | WIRELESS_STAT_MULTICAST_TRANSMIT |
| Transmitted Error Count | The count of packets AP fails to transmit | WIRELESS_STAT_TX_ERROR |
| Transmitted Total Retry Count | The count of packets AP retries to transmit | WIRELESS_STAT_TX_TOTAL_RETRIES |
| Transmitted Multiple Retry Count | The count of packets AP transmits more than once. | WIRELESS_STAT_TX_TOTAL_RETRIES |
| Received Fragment Count | The count of packets AP received without errors | WIRELESS_STAT_GOOD_RECEIVE |
| Multicast Received Frame Count | The count of multicast packets AP received without errors | WIRELESS_STAT_MULTICAST_RECEIVE |
| Received Frame FCS Error Count | The count of packets fail to check sequence | WIRELESS_STAT_RX_FCS_ERROR |
| Received Frame Duplicate Count | The count of same packets AP received more than once | WIRELESS_STAT_RX_DUPLICATES |
| Ack Rcv Failure Count | The count of ACK packets AP fails to receive | WIRELESS_STAT_ACK_RECEIVE_ERROR |
| Wep Excluded Frame Count | The count of received encrypted packets that is not match our encryption policy | WIRELESS_STAT_RX_WEP_EXCLUDED |
| Wep ICV Error Count | The count of received encrypted packets that decrypted with error | WIRELESS_STAT_RX_DECRYPT_CRC_ERROR |