DWL-2100AP

Date: March 8, 2004

| Product name: | 802.11g Wireless Access Point |
| --- | --- |
| Product Group & Type: | Wireless Access Point |
| Revision: | A1/A2 |
| Available: |  |
| Picture: |  |
| Why Sell This Product: | - Eventual replacement for DWL-2000AP. This DWL-2100AP is designed and produced by Alpha, while the DWL-2000AP is outsourced. The DWL-2100AP uses the same Atheros chipset as DWL-2000AP, but emphasizes more network management functions to be supported by Alpha. Selling Points - 108Mbps Super G high speed. - WAP security & management functions. |
| Keyfeatures: | - 802.11g 2.4GHz standard - Up to 108Mbps (Turbo mode) high-speed transmission - 10/100BASE-TX port for connection to Ethernet or DSL/cable modem - Compatible with existing 802.11b wireless devices - Built-in DHCP server for multiple user access - 5 operation modes: AP, AP client, point-to-point bridge, point-to-multi-point bridge, repeater - 64/128/152-bit WEP data encryption - WPA security with 802.1x RADIUS user authentication - Web-based configuration & management |
| Technical Details: | Standards - IEEE 802.11g, 802.11b wireless LAN - IEEE 802.3 Ethernet/IEEE 802.3u Fast Ethernet Device Management - Web-Based: Internet Explorer v6 or later; Netscape Navigator v6 or later; or other Java- enabled browsers - DHCP Server and Client Wireless Data Rates (With Automatic Fallback) 54 Mbps, 48 Mbps, 36 Mbps, 24 Mbps, 18 Mbps, 12 Mbps, 11 Mbps, 9 Mbps, 6 Mbps, 5.5 Mbps, 2 Mbps, 1 Mbps Security - 64/128/152-bit WEP data encryption - 802.1x authentication - WPA: Wi-Fi Protected Access (TKIP, MIC, IV Expansion, Shared Key Authentication) Media Access Control CSMA/CA with ACK Wireless Frequency Range - 802.11b: 2400 to 2483.5MHz ISM band - 802.11g: 2400 to 2483.5MHz ISM band RF Modulation Schemes - 802.11b: DQPSK, DBPSK and CCK - 802.11g: BPSK, QPSK, 16QAM, 64QAM, OFDM Receive Sensitivity (802.11b) @ 8% PER (packet error rate) - 11 Mbps: -83 dBm - 2 Mbps: -89 dBm Receive Sensitivity (802.11g) Frame: 1000byte PDUs, @ 10% PER (packet error rate) - 54 Mbps: -66 dBm - 48 Mbps: -71 dBm - 36 Mbps: -76 dBm - 24 Mbps: -80 dBm - 18 Mbps: -83 dBm - 12 Mbps: -85 dBm - 9 Mbps: -86 dBm - 6 Mbps: -87 dBm Transmit Output Power (802.11b) 18 dBm @ 11Mbps, 5.5Mbps, 2Mbps, 1Mbps Transmit Output Power (802.11g) - 14 to 15 dBm @ 54Mbps, 108Mbps, - 14 to 16 dBm @ 48Mbps - 16 to 18 dBm @ 36Mbps, 24Mbps, 18Mbps, 12Mbps, 9Mbps, 6Mbps Antenna - 2dBi Gain detachable dipole antenna with reverse SMA connector - 2dBi Gain internal antenna Wireless Operating Range * - Indoors: Up to 100 m (328 feet) - Outdoors: Up to 400 m (1,312 feet) * Environmental factors may adversely affect operation range. LEDs Power - WAN - LAN (10/100Mbps) - WLAN Physical & Environmental Power Input - DC 5V, 2.0A - Through external power adapter Power Consumption 5 watts (max.) Dimensions 142 (L) x 109 (W) x 31 (H) mm Operating Temperature 0 to 55 C Storage Temperature -20 to 75 C Humidity 95% maximum (non-condensing) EMI Certification - FCC Class B - CE Class B |
| EMI/EMC Report | FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No. ACN052202838  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others |
| Safety |  TUV No.  UL No.  CSA No.  Others |
| Packaging Style |  Clamshell  White Box  Gift Box  Bulk  Others |
| Packaging Contents: | DWL-2100AP wireless AP, QIG,AC Power Adapter,Ethernet Cable,manual and AP manager on CD |
| UPC Code: | BDWL2100..A1 790069-263958; DWL2100BEUA1 790069-266942. DWL2100EEUA1 790069-266959; DWL2100NEUA2 790069-266966. CDWL2100..A2 790069-266997; DWL2100ANAA2 790069-266935. DWL2100ENAA2 790069-266973; NDWL2100..A2 790069-267000. DWL2100IS.A2 790069-266980; TDWL2100..A2 790069-267017. |
| Units Per Carton: | 10 |
| Cartons Per Pallet: | Air: 24, Sea: 42 |
| Dimensions and Weight: | Box: 275 x 210 x 65 mm, 0.70 kg Carton: 435 x 354 x 294 mm, 7.63 kg Pallet: (by Air) 120 x110 x 135.6 cm/Per Pallet A , 198.12 Kg 120 x110 x 136.6 cm/Per Pallet C , 198.12 Kg 110 x120 x 137.1 cm/Per Pallet E , 201.62 Kg (by Sea) 120 x110 x 197.4 cm/Per Pallet A, 335.46 Kg 120 x110 x 198.4 cm/Per Pallet C, 335.46 Kg 110 x120 x 198.9 cm/Per Pallet E, 338.96 Kg INTRASTAT weight: kg (Box) Packing material weight (Box): - Paper: kg - Plastic: kg - Mixed: kg |