# Page 1

This product can be set up using any current web browser, i.e., Internet Explorer 6 or Netscape Navigator 6.2.3.

D-Link AirPremier™ 802.11g/2.4GHz/108Mbps Wireless Access Point

## Before You Begin

It’s best to use a computer (with an Ethernet adapter) that is connected to a switch for configuring the DWL-2200AP. The default IP address for the DWL-2200AP is 192.168.0.50 with a Subnet Mask of 255.255.255.0. You will need to assign your computer a Static IP address within the same range as the DWL-2200AP’s IP address for the purpose of configuring the DWL-2200AP. See the Appendix A if you need assistance assigning a Static IP address for your network adapter.

## Check Your Package Contents

DWL-2200AP AirPremier™ Wireless Access Point

CD-ROM with Manual

Ethernet (CAT5e UTP/Straight-Through) Cable

PoE Base Unit

Power Cord

Power Adapter

Using a power adapter with a different voltage rating will damage and void the warranty of this product.

If any of the above items are missing, please contact your reseller.

©2004 D-Link Systems, Inc. All rights reserved. Trademarks or registered trademarks are the property of their respective holders. Software and specifications subject to change without notice.

### Page 2

## Connecting the DWL-2200AP to Your Network

There are two methods of powering the DWL-2200AP. The power adapter can plug directly into the unit or plug into the PoE (Power over Ethernet) base unit. For directions on using PoE, please refer to Appendix B.

A.	Connect the power adapter to the receptor at the back panel of the DWL-2200AP. Plug the other end of the power adapter to a wall outlet or power strip. The Power LED will turn ON to indicate proper operation.

B.	Insert one end of the cable to the Ethernet port on the back panel of the DWL-2200AP and the other end of the cable to an Ethernet Broadband Router (i.e., D-Link DI-604) or Switch (i.e., D-Link DSS-5+). Note: You also have the option of connecting the DWL-2200AP directly to the computer that will be used for configuration. The LAN LED light should illuminate to indicate a proper Ethernet connection. (Note: The Ethernet Port on the DWL-2200AP is Auto-MDI/MDIX. This allows you to use a straight-through or crossover Ethernet cable to connect 	to the Ethernet port on the DWL-2200AP.)

C.	The DWL-G650 AirPlus Xtreme G™ Wireless Cardbus Adapter and the DWL-G520 AirPlus Xtreme G™ Wireless PCI Adapter will connect, out of the box with the DWL-2200AP, using their default wireless settings. Computers with 802.11b	wireless adapters can also connect to the DWL-2200AP.

### Page 3

Connecting the DWL-2200AP to Your Network (continued)

Front Panel

LAN LED

A solid green light indicates a good network connection. The light will blink when there is activity on the LAN.

Power LED

The Power LED will light up solid green when properly connected to a valid power source.

WLAN LED

The Wireless LAN LED will blink to indicate wireless network activity.

Rear Panel

Antenna Connector

Attach the wireless antenna to this connector.

LAN Port

Connects to a computer, a switch, router, or PoE base unit using an Ethernet cable.

Reset Button

Pressing this button restores the Access Point to its original factory default settings.

Receptor

Used to connect Power Adapter directly to unit.

### Page 4

Connecting the DWL-2200AP to Your Network (continued)

When you have completed the steps in this Quick Installation Guide, your connected network should look similar to this:

Page 5

## Using the Setup Wizard

Open your Web browser and enter http://192.168.0.50

The log-in pop-up screen will appear.

Type admin for the username and leave the password field blank.

Click OK

Once you have logged in, the Home screen will appear.

Click Run Wizard

Page 6

Using The Setup Wizard (continued)

You will see the following screen.

Click Next

Step 1 - Set up your new password. You have the option to establish a password.

Click Next

Page 7

Using The Setup Wizard (continued)

Step 2 - Wireless Setup

The default wireless settings are:

SSID = default

Channel = 6

You can change these settings to match an existing wireless network.

Click Next

Step 3 - Encryption

The DWL-2200AP is capable of three levels of wireless encryption: 64-bit, 128-bit, and 152-bit. By default encryption is disabled. You can enable encryption for secure wireless communication.

Click Next

### Page 8

Using The Setup Wizard (continued)

Click Restart

Click Close

### Page 9

## Your Setup is Complete!

You will be returned to the Home tab.

Click to Exit

For additional settings or information, refer to the Advanced, Tools, or Status tabs on the web-management interface; or to the manual located on the CD-ROM.

### Page 10

## Appendix A - Assigning a Static IP Address

To assign a Static IP Address on the computer that you are using to configure the DWL-2200AP, follow the steps for the appropriate operating system:

Windows XP:

Go to Start, click on Control Panel double-click on Network Connections,  right-click on Local Area Connections  and select Properties.

Highlight Internet Protocol (TCP/IP)

Click Properties

Input a static IP address in the same range as the DWL-2200AP’s IP address of 192.168.0.50. Each computer/device must have a unique IP address.

Select Use the following IP address

IP address: 192.168.0.51

Subnet mask: 255.255.255.0

Click OK

### Page 11

Appendix A - Assigning a Static IP Address (continued)

Windows 2000:

Go to Start > Settings > Network and Dial-up Connections. Double-click on the Local Area Connection associated with the Network Adapter installed on your system and select Properties.

Click Internet Protocol (TCP/IP)

Click Properties

Input a static IP address in the same range as the DWL-2200AP’s IP address of 192.168.0.50. Each computer/device must have a unique IP address.

Select Use the following IP address

IP address: 192.168.0.52

Subnet mask: 255.255.255.0

Click OK

### Page 12

Appendix A - Assigning a Static IP Address (continued)

Windows Me & 98SE:

Go to Start > Settings > Control Panel > Double-click on the Local Area Connection associated with the Network Adapter installed on your system > select Properties

Click Internet Protocol (TCP/IP)

Click Properties

Input a static IP address in the same range as the DWL-2200AP’s IP address of 192.168.0.50. Each computer/device must have a unique IP address.

Select Specify IP Address

IP address: 192.168.0.52

Subnet mask: 255.255.255.0

Click OK

### Page 13

Appendix A - Assigning a Static IP Address (continued)

Mac OS X:

Go to the Apple Menu and Select System Preferences.

Click on Network

Select Built in Ethernet from the Show pull-down menu

Select Manually in the Configuration pull-down menu

Input a static IP address in the same range as the DWL-2200AP’s IP address of 192.168.0.50. Each computer/device must have a unique IP address.

IP address: 192.168.0.28

Subnet mask: 255.255.255.0

Click on Apply Now

### Page 14

Appendix B - Using PoE (Power over Ethernet)

•	Step 1: Connect one end of an Ethernet cable (included with your package) to the LAN port on the DWL-2200AP and the other end of the Ethernet cable to the port labeled P+DATA OUT on the PoE base unit.

•	Step 2: Connect another Ethernet cable from the DATA IN port on the PoE base unit to your router/switch or to a PC.

•Step 3: Attach the power adapter to the connector labeled POWER IN on the PoE base unit. Attach the power cord to the power adapter and into an electrical outlet.