Problems Resolved:

- Fix the issue of output power for EU.

- In MSSID+WDS function, change the rule of dropping packets without vlantag. when vlan state enabled (WDS with AP mode), the AP would forwwad the packets from WDS without Vlantag.

- Dell notebook PC (with Broadcom ethernet adapter) still could manage the AP without vlantag, when management VLAN enabled.

- In MSSID+WDS function, the rule of dropping packets without vlantag, does not work well, when vlan state enabled.

- Primary SSID packets to Ethernet with VLAN tag function could not take effect.

- Change the username length from 12 characters to 64 characters

- Multi-SSID stable problem，when enable Multi-SSID and set WPA-PSK authentication, and input wrong pass phrase for several days, then input correct pass phrase, station could not connect to the ap.

- When user login with wrong user/password, they still can access some web pages

- Close auto saving function for configuration file, saving update once per min.

- Delete some useless messages of deauthentications, which are virtue deauthentication by ap for special use.

- IOT issue, If enable the superG and WMM, some wireless card can not communicate with the AP when the Authentication is WPA and Cipher type is TKIP

- Fix the Password issue under “Administrator Settings” item, when saving the settings of this page without changing password, and old password is not blank, it should pop-up an error

- Fix the bug that GSM/Wi-Fi cell phone always be disconnected when it connects to secondary SSID

- Fix the bug of login, we can login in with “user” and “password”

- When using some special characters in name or password, user will can't login successfully

Enhancements:

- Modify the Data Rate Control

- Enable Multicast Rate Control on CLI

- Add MSSID+WDS function.

- Add management VLAN, which let administrator can manage the AP through special vlan id.

- Add Primary SSID packets to Ethernet with VLAN tag function

- Add data rate control .function.

- Add AP client function for AP mode