# DWL-3200 AP Firmware Release Note

Firmware: v2.40rc167

Hardware: B1

Date: June. 22, 2007

Problems Resolved:

- Updating firmware from APMII will failed caused by memory size.

- [WEB] Close two back door accounts. (Two user name and password - demo/demo and user/password, can login WEB page.)

- [WEB&APMII] Fix the issue that System Time on home page is one hour later than Local Time on SNTP page when Daylight Saving Time is enabled.

- [FW] Second radius settings have not sent to APM/APMII/DV in config file.

- [FW] Can’t save data rate control setting with APM/APMII/DV

- [FW]IAPP roaming can’t send MOVE-NOTIFY packets.

- [FW]AP can’t send coldstart trap, when user push the reset button.

- [FW]AP hadn’t sent trap message,when CPU loading full.

- [FW]AP crashed,when authentication type is WPA.

- [WEB]User set SNMP server’s IP failed.

- [FW]MSSID cann’t response the probe request,when STA scaning.

- [FW]Multi-SSID stable problem: when enable Multi-SSID with WPA-PSK, AP will stop services after several days

- [WEB]When user login with wrong user/password, they still can access some web pages

- [WEB]The WMM's status of Primary SSID cann't display rightly in Multi-SSID webpage

- [WEB]SNTP time server error

Enhancements:

- Support APM-II

- Support WMM to 802.1p mapping, VLAN tagged packet forwarding(disabled).

- Multicast packet control, from LAN to WLAN

- On WEB UI, add Data Rate Control (refer to WiFi manager).

- Support 802.11MIB

- Support MIB-II (RFC-1213)

- AP status on UI (also on MIB)

  - CPU and Memory utilization

  - Statistics for Ethernet TX and RX,bytes and packets

- Secondary RADIUS server setting

- Trap server, settings(catagorize the types of traps)

- Enable/Diabe ping

- Rouge AP protection

- support IAPP(802.11f) roaming

==============================================================