DWL-3260AP_QIG

| English | Translation |
| --- | --- |
| Page1 |  |
| Quick Install Guide |  |
| Managed Wireless Access Point |  |
| Page2 |  |
| System Requirements |  |
| Minimum System Requirements: • CD-ROM Drive • Computers with Windows, Macintosh, or Linux-based operating systems • Installed Ethernet Adapter Internet Explorer version 6.0 or Netscape Navigator version 7.0 and above. |  |
| Package Contents |  |
|  D-Link AirPremierTM DWL-3260AP Managed Wireless Access Point |  |
| Power over Ethernet base unit |  |
|  Power Adapter-DC 48V, 0.4A |  |
| Power Cord |  |
|  Manual and Warranty on CD |  |
|  Quick Installation Guide |  |
| Ethernet Cable |  |
| Mounting Plate |  |
| Note: Using a power supply with a different voltage than the one included with the DWL-3260AP will cause damage and void the warranty for this product. |  |
| If any of the above items are missing, please contact your reseller. |  |
| Page3 |  |
| Installing the Hardware |  |
| status LED |  |
| reset button |  |
| external antenna |  |
| Notice: DWL-3260AP has embedded antenna inside,if you need to use external antenna,please turn the switch button to Ext. and attach the antenna. |  |
| Page 4 |  |
| Connecting the Ethernet and Power Cables |  |
| The access point receives power through the Ethernet cable or an external power adapter. And also, this product intended only for installation in Environment A as defined in IEEE 802.3af, Power over Ethernet (PoE). |  |
| The access point power options: |  |
| Option 1: Connect with a D-Link wireless switch or other 802.3af PoE compliant switch. |  |
| Option 2: Use the PoE Base Unit to connect to a switch that lacks PoE capabilities. |  |
| Option 3: Connect to available power using the included power adapter . Note: This product and all interconnected equipment must be installed indoors, including all associated LAN connections. |  |
| Page5 |  |
| Physical Installation for Your Access Point:(optional) |  |
| Junction Box Installation: 1.Attach the bracket to the junction box. 2.attach the DWL-3260AP to the mounting bracket. |  |
| Solid Wall or Ceiling Installation: 1.Mount the bracket on the appropriate position on the wall or Ceiling. 2.Mount the AP to the bracket on the wall or ceiling. |  |