| Firmware Version: | 3.21rc166 |
| --- | --- |
| Published Date: | 2009/7/3 |

Content:

Revision History and System Requirement:	2

Upgrading Instructions:	2

Upgrading by using Web-UI	2

New Features:	2

Changes of MIB & D-View Module:	3

Problems Fixed:	3

Known Issues:	3

Related Documentation:	3

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| v3.21rc165 | 2009/7/2 | DWL-7700AP | B1,B2,C1 |
| V3.20rc156 | 2008/8/18 | DWL-7700AP | B1,B2,C1 |

# Upgrading Instructions:

## Upgrading by using Web-UI

- Connect a work station to the LAN port of Access Point.

- Open the web browser of work station and entering the IP address of the AP. The system

default IP address is 192.168.0.50.

- Enter administrator’s username and password, user name is “admin” and leave

the password blank by default.

To update the AP's firmware, click Tool > Firmware and SSL Certification Upload.

# New Features:

| Fireware Version | New Features |
| --- | --- |
| V3.21 | - Add the new flash chip support |
| V3.20 | - Add Administration VLAN feature - Add Primary SSID with VLAN tag. - Add Secondary (backup) Radius Server for WPA (2) Enterprise settings. - Support Priority by SSID. - Support Microsoft NAP feature. - Support IGMP snooping v1/v2. - Add Rogue AP detection function. - Support Acct-Interim-Interval attribute for accounting function. - Add system and wireless sub trap group. User can also enable or disable the trap categorization. - Support to administrate AP with WLAN (Default enable). - Support four management IP range. - Add CLI command to input SNMP Location. - Add Ping control setting. - Add WMM control on each MSSID. - Support AP Manager II v2. - Support MSSID over WDS. - Support IAPP. - Add wireless station linkup and link down trap. |

# Changes of MIB & D-View Module:

None

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V3.21 | None |
| V3.20 | - User will fail to login to the management web UI via IE 7.0 if silverlight was installed. - MSSID will be modified if primary SSID security is set to 802.1x. - When change primary SSID settings, sometimes MSSID will also be changed. |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |
| V3.21 | None |
| V3.20 | None |

# Related Documentation:

- DWL-7700AP User Manual v3.20