Pls. translate below English and keep original English words.

This product can be set up using any current web browser, i.e., Internet Explorer 6 or Netscape Navigator 6.2.3.

Wireless Range Extender

It’s best to use a computer (with an Ethernet adapter) that is connected to a switch for configuring the DWL-800AP+. The default IP address for the DWL-800AP+ is 192.168.0.30 with a Subnet Mask of 255.255.255.0. You will need to assign your computer a Static IP address within the same range as the DWL-800AP+’s IP address for the purpose of configuring the DWL-800AP+. See the Manual on the CD-ROM if you need assistance in assigning a Static IP address for your network adapter.

DWL-800AP+ Wireless Range Extender

Connecting The DWL-800AP+ Wireless Range Extender To Your Network

- First, connect the power adapter to the receptor at the back panel of  the DWL-800AP+ and then plug the other end of the power adapter to a wall outlet or power strip. The Power LED will turn ON to indicate proper operation.

- Using a Ethernet cable, insert one end of the cable to the LAN port on the back panel of the DWL-800AP+ and the other end of the cable to a switch. Note: You also have the option of connecting the DWL-800AP+ directly to the computer that will be used for configuration.  The Link LED light will illuminate to indicate a proper Ethernet connection. (Note:The LAN Port on the DWL-800AP+ is Auto-MDI/MDIX.  Meaning you can use a straight-through or crossover-cable to the LAN Port.)

- Computers with 802.11b wireless adapters such as the DWL-650+ AirPlus Wireless Cardbus Adapter and the DWL-520+ AirPlus Wireless PCI Adapter can connect to the DWL-800AP+.  By default, the DWL-800AP+ is set in repeater mode and will extend the wireless signal from a wireless gateway or access point.

Step 1 - Set up your new password. You have the option to establish a password.

Step 2 - Wireless Setup

By default the DWL-800AP+ is set to Repeater mode.

To repeat the wireless signal, enter the 12 digit Ethernet MAC Address of the wireless router or access point.

Step 3 - Encryption

The DWL-800AP+ is capable of three levels of wireless encryption - 64-bit, 128-bit, and 256-bit. By default the encryption is disabled. You can change the encryption settings for more secure wireless communication.

You will be returned to the Home tab.  Please close all internet browser windows.

Connecting to the DWL-800AP+

Using the AirPlus Wireless Utillity included with the AirPlus Adapters, you can browse and connect to the DWL-800AP+.

The MAC address of the DWL-800AP+ is listed on the product sticker on the bottom of the unit.

The Link Info page of the AirPlus utility will show the connection status between the DWL-800AP+ and AirPlus wireless adapter.