| Wireless Card #### Tested/Verified Report Product Model; DWL-G122. Product Description; USB. Driver Version; 1.1.3.0 (5 OSs). Utility Version; C1: V3.10 B1: V2.04. Driver Version; C1: Driver : V1.0.1.0 (6OS) B1:Driver V2.0.1.0 (Win 2K/XP) Driver: V2.1.0.0 (Win 98/ME) Driver: V2.0.3.0 (XP-64-bit). Build Version; 3.3.2.61614. Setup Version; S0032. H.W Version; C1 /B1. PCB Version; 3A1. Product Key Component; C1: Ralink RT2571W+ RT2528 B1: Ralink RT2571W+RT2526. |
| --- |
| Approved by | Checked by | Department | Prepared by |
| Margaret Lee | Margaret Lee | TLD2 | Daby ,Jmmy |

Plan Revision History

| Rev. | Date | Author | Reason for Changes |
| --- | --- | --- | --- |
| A1 | 2004/09/01 | Daby | First version. |
| A2 | 2004/09/20 | Emily | Update Security section |
| A3 | 2004/09/29 | Andy | Add firewall in Win XP with SP2 function |
| A4 | 2004/11/22 | Daby | Update PC/Note Book list |
| A5 | 2004/12/01 | Daby | Update Channel function |
| A5.1 | 2004/12/07 | Emily | Update Basic function for conceptronic UI |
| A6 | 2005/03/09 | Daby | Add Atheros Utility |
| A7 | 2005/03/10 | Daby | Add WPA2 Security function |
| A8 | 2005/05/11 | Daby | Add WMM (Wi-Fi Multimedia) function |
| A9 | 2005/09/30 | Daby | Add JumpStart function |
| A10 | 2005/11/01 | Daby | Add XP_64Bit function |
| A11 | 2006/01/04 | Daby | Add in Super Mode Endurance test <Item5-3> |
| A12 | 2006/12/04 | Daby | Add in Super Mode to copy big file and run “Rescan” from utility <Item 2-12-1> # 60103SM01 |
| A13 | 2006/01/10 | Daby | Add in NTFS file system in Win XP with SP2 <Item 2-5> # 60105SM01 |
| A14 | 2006/02/21 | Daby | Add in domain login by WPA-TLS and WPA2-TLS |
| A15 | 2006/06/12 | Daby | Add JumpStart function <Item 6-5-1, 6-5-2> |
| A16 | 2006/06/13 | Daby | Add Channel function <2-4-1, 2-4-2, 2-4-3> |
| A17 | 2006/10/24 | Daby | Add function “Driver Modifying Protection with a Normal User” <2-17> |

Product Release History

| Date | Release Version | RD | Solved Bug Description |
| --- | --- | --- | --- |
| 2004/10/24 | Utility Version:v3.00 Driver Version:1.00.00.0000 Build Version: 3.3.1.50928 Reg Domain: FCC Firmware: N/A Setup Version: S0002 | Lily Hsu | Test All Driver (Win2K/XP/98 /ME/XP_64BIT) |
| 2004/10/04 | Utility Version:v3.00 Driver Version:1.00.10.0000 Build Version: 3.3.1.50928 Reg Domain: FCC Firmware: N/A Setup Version: S0004 | Lily Hsu | - Update C1 driver to 1.0.1.0 - Solved transmit can’t work issue. - Solved DI-624/DI-784 WPA-PSK issue |
| 2005/11/17 | Utility Version:v3.00 Driver Version:2.00.01.0000 Build Version: 3.3.1.51111 Reg Domain: FCC Firmware: N/A Setup Version: S0005 | Lily Hsu | Fix system hang up under OS and update F/W, driver not change |
| 2005/11/18 | Utility Version:v3.00 Driver Version:2.00.01.0000 Build Version: 3.3.1.51111 Reg Domain: FCC Firmware: N/A Setup Version: S0006 | Lily Hsu | Fix copy file path only in driver path |
| 2005/11/20 | Utility Version:v3.00 Driver Version:2.00.01.0000 Build Version: 3.3.1.51111 Reg Domain: FCC Firmware: N/A Setup Version: S0007 | Lily Hsu | Ralink change it's folder path for RT73.bin to Windows\System. But for avoiding more problems, RT73.bin would be copy to more path (Windows\System32\Drivers and Windows\System) for Win98/ME. |
| 2005/11/21 | Utility Version:v3.00 Driver Version:2.00.01.0000 Build Version: 3.3.1.51111 Reg Domain: FCC Firmware: N/A Setup Version: S0008 | Lily Hsu | Copy 9x driver to system32\drivers, just like Atheros do. |

| Date | Release Version | RD | Solved Bug Description |
| --- | --- | --- | --- |
| 2005/11/23 | Utility Version:v3.00 Driver Version:2.00.01.0000 Build Version: 3.3.1.51123 Reg Domain: FCC Firmware: N/A Setup Version: S0009 | Lily Hsu | For Ralink solution, adhoc profile should update with adhoc channel |
| 2006/08/14 | Utility Version:v3.10 Driver Version:1.1.1.0 Build Version: 3.3.1.60719 Reg Domain: ETSI Firmware: N/A Setup Version: S0012 | Lily Hsu | 1. Support Hardware B1 and C1 2. Update G122 C1 driver to v1.1.1.0 3. Wireless mode setting issue |
| 2006/10/24 | Utility Version v3.10 Driver Version: 1.1.3 Build Version:3.3.1.60922 Setup Version:S0015 Reg Domain: FCC Firmware:1.3 | Lily Hsu | 1. Fix PSP Support Function. 2. Update Driver to 1.1.3.0 |
| 2006/11/20 | Utility Version v3.10 Driver Version: 1.1.3.0 Build Version:3.3.1.61116 Setup Version:S0016 Reg Domain: FCC Firmware:1.3 | Lily Hsu | 1.Add WHQL for V1.1.3.0 2.Fix popup configuration page : switch authentication type "-PSK" to "Shared", Key1~4 Grayed. 3.Link information : "Authenticated"->"herniated". Use nonbrand style rule. |
| 2006/12/26 | Utility Version v3.10 Driver Version: 1.1.3.0 Build Version:3.3.1.61116 Setup Version:S0018 Reg Domain: FCC Firmware:1.3 | Lily Hsu | - Update Setup Version to S0018 - Update service again for Ch1 poor throughput. While in Infrastructure mode, service automatically turn off PSP Xlink. |
| 2007/02/02 | Driver Version: 1.0.1.0 | Lily Hsu | Add Vista driver v1.0.1.0. |
| 2007/04/06 | Utility Version v3.10 Driver Version: 1.1.3.0 Build Version:3.3.1.61116 Setup Version:S0018 Reg Domain: FCC Firmware:1.3 | Lily Hsu | [S0023] 1.Modify multi language string table. |

| 2007/04/06 | (1) DWL-G122.B1 Utility version : V2.04 Driver: V2.0.1.0 (Win 2K/XP) Driver: V2.1.0.0 (Win 98/ME) Driver: V2.0.3.0 (XP-64-bit) (2) DWL-G122.C1 Utility version : V3.10 Driver : V1.0.1.0 (5OS) | Lily Hsu | [S0025][15ML] 1.Reverse driver to V1.0.1.0. 2.Fix XP64bit fail to install driver. |
| --- | --- | --- | --- |
| 2007/04/06 | (1) DWL-G122.B1 Utility version : V2.04 Driver: V2.0.1.0 (Win 2K/XP) Driver: V2.1.0.0 (Win 98/ME) Driver: V2.0.3.0 (XP-64-bit) (2) DWL-G122.C1 Utility version : V3.10 Driver : V1.0.1.0 (5OS) | Lily Hsu | [S0026] 1.Update XP64bit driver to V1.0.1.0. |
| 2007/04/06 | (1) DWL-G122.B1 Utility version : V2.04 Driver: V2.0.1.0 (Win 2K/XP) Driver: V2.1.0.0 (Win 98/ME) Driver: V2.0.3.0 (XP-64-bit) (2) DWL-G122.C1 Utility version : V3.10 Driver : V1.0.1.0 (5OS) | Lily Hsu | [S0027] 1.Couldn't load RT25U98.sys in Device Manager.[Win98SE/ME] |
| 2007/04/06 | (1) DWL-G122.B1 Utility version : V2.04 Driver: V2.0.1.0 (Win 2K/XP) Driver: V2.1.0.0 (Win 98/ME) Driver: V2.0.3.0 (XP-64-bit) (2) DWL-G122.C1 Utility version : V3.10 Driver : V1.0.1.0 (5OS) | Lily Hsu | [S0028] 1.Change 122.B1 driver to V2.1.0.0. |
| 2007/04/09 | (1) DWL-G122.B1 Utility version : V2.04 Driver: V2.0.1.0 (Win 2K/XP) Driver: V2.1.0.0 (Win 98/ME) Driver: V2.0.3.0 (XP-64-bit) (2) DWL-G122.C1 Utility version : V3.10 Driver : V1.0.1.0 (5OS) | Lily Hsu | [S0029] 1.[Greek]Utility and Wizard fail to show because the installer instance path is greek. 2.Add V1.0.1.0 Windows XP64bit WHQL file for DWL-G122.C1. |

| 2007/04/11 | (1) DWL-G122.B1 Utility version : V2.04 Driver: V2.0.1.0 (Win 2K/XP) Driver: V2.1.0.0 (Win 98/ME) Driver: V2.0.3.0 (XP-64-bit) (2) DWL-G122.C1 Utility version : V3.10 Driver : V1.0.1.0 (5OS) | Lily Hsu | [S0030] 1.Reverse Dr71WU.inf in \win64 folder back to WHQL version. |
| --- | --- | --- | --- |
| 2007/04/13 | (1) DWL-G122.B1 Utility version : V2.04 Driver: V2.0.1.0 (Win 2K/XP) Driver: V2.1.0.0 (Win 98/ME) Driver: V2.0.3.0 (XP-64-bit) (2) DWL-G122.C1 Utility version : V3.10 Driver : V1.0.1.0 (5OS) | Lily Hsu | [S0031] 1.Fix Wizard wep profile cause Utility hang issue. 2.Found Installer Language List (Normal Initialization dialog) cause Intel 915GM/GMS, 910GML Express Chipset Family Driver Crash. Only by update intel driver would solve this issue. note:This setup still use normal language list style. The driver v1.0.1.0 has USB1.1 compatibility issue. |
| 2004/04/16 | (1) DWL-G122.B1 Utility version : V2.04 Driver: V2.0.1.0 (Win 2K/XP) Driver: V2.1.0.0 (Win 98/ME) Driver: V2.0.3.0 (XP-64-bit) (2) DWL-G122.C1 Utility version : V3.10 Driver : V1.0.1.0 (5OS) | Lily Hsu | [S0032] 1.Fix keyboard selection not be stored in Wizard site survey list. 2.Fix missing character in tooltiptext. [Root Cause: Couldn't include DDK2660 and DDK6000 in the same compiler] |

Product Test History

| Release Date | Lab-Out Driver Version | Tester | Note |
| --- | --- | --- | --- |
| 2006/10/24 | DWL-G122 C1: Utility Version v3.10 Driver Version: 1.1.3 Build Version:3.3.1.60922 Setup Version:S0015 Reg Domain: FCC Firmware:1.3 | Jerry, Daby | We started this test at 10/24 and 4 working days were used to finish it. This test was held for the driver upgrade from 1.1.2 to 1.1.3 because of fixing the PSP function support bug. The driver version 1.1.3 supports 5 Windows systems (including Windows 98SE, Windows ME, Windows 2000, Windows XP and Windows XP x64 Edition). For this driver upgrade, we carried out the full functions test and find 2 bugs. |
| 2006/11/07 | DWL-G122 C1: Utility Version v3.10 Driver Version: 1.1.3.0 Build Version:3.3.1.61116 Setup Version:S0015 Reg Domain: FCC Firmware:1.3 | Jerry, Daby | At 11/07, we rechecked and fixed the bugs, #2, #4, #5, #6 and #7. |
| 2006/11/20 | DWL-G122 C1: Utility Version v3.10 Driver Version: 1.1.3.0 Build Version:3.3.1.61116 Setup Version:S0016 Reg Domain: FCC Firmware:1.3 | Jerry, Daby | At 11/20, we got a new driver version (UI: v3.10, driver: 1.1.3.0, and build: 3.3.1.61116). This driver will be the newest WHQL certificated XP and XP64 driver. We checked the .cat files and driver installation/un-installation for 5 Windows Oss. Besides, because the R/D fixed three issues in this version, we hence also checked them in this test. The three issues are shown below: - Add WHQL file for driver V1.1.3.0. - Fix popup configuration page : switch authentication type "-PSK" to "Shared", Key1~4 Grayed Fix link information: "Authenticated"->"herniated". Use nonbrand style rule: Pass |

| 2006/11/22 | DWL-G122 C1: Utility Version v3.10 Driver Version: 1.1.3.0 Build Version:3.3.1.61116 Setup Version:S0016 Reg Domain: FCC Firmware:1.3 | Daby , Jerry | At 11/22, we find a bug about the driver un-installation error and add it into the test report. |
| --- | --- | --- | --- |
| 2006/12/28 | DWL-G122 C1: Utility Version v3.10 Driver Version: 1.1.3.0 Build Version:3.3.1.61116 Setup Version:S0018 Reg Domain: FCC Firmware:1.3 | Daby , Jerry | At 12/28, we rechecked D-Link feedback the Ch1 poor throughput issue. We found that the poor throughput is related to the anti-static label. We found the throughput would become well when we remove the anti-static label from DUT. The test throughputs were between 20 ~ 21 Mbps. Besides, we also found a new issue related to the WPA2-PSK connection problem. RD has rechecked this issue and they are still finding the cause of this problem. |
| 2007/1/8 | Utility Version v3.10 Driver Version: 1.1.3.0 Build Version:3.3.1.61116 Setup Version:S0018 Reg Domain: FCC Firmware:1.3 | Daby , Jerry | At 2007/01/08, RD rechecked this problem in their environment, but they could not duplicate this issue again. We therefore tried to duplicate this issue again in the same environment, but we also could not duplicate it again by trying several times. We hence close the Bug ID #10 temporarily. |
| 2007/01/24 | Driver Version: 2.0.1.0 (B1) 1.0.1.0 (C1) Utility Version: 2.04 (B1) 3.10 (C1) Setup Version: S0023 | Daby , Jerry Gloria | Test Language: 15 countries. |
| 2007/2/15 | DWL-G122 C1: Utility Version v3.10 Driver Version: 1.1.3.0 Build Version:3.3.1.61116 Setup Version:S0018 Reg Domain: FCC Firmware:1.3 | Daby , Jerry | At 2007/01/15, we rechecked and fixed the Bug ID #1, security transmission issue in WPA-EAP with Cipher-Auto mode, in setup version S0018. The Bug ID #3 is an OS issue of Windows 2004 SP4, and we therefore close it. Left bug#8. |

| 2007/2/2/1 | (1) DWL-G122.C1 Driver : V1.0.1.0 (Vista) | Gloria, Abyss | The test was started at 2007/2/1 and finish Vista driver test.. |
| --- | --- | --- | --- |
| 2007/4/16 | (1) DWL-G122.B1 Utility version : V2.04 Driver: V2.0.1.0 (Win 2K/XP) Driver: V2.1.0.0 (Win 98/ME) Driver: V2.0.3.0 (XP-64-bit) (2) DWL-G122.C1 Utility version : V3.10 Driver : V1.0.1.0 (5OS) | Daby , Jimmy | Fixing : Can’t load WinXP64 Driver problem and double checking Multi language and UI |

Test Result

This driver package is similar to latest release, but added Vista driver.

For hardware version B1, the driver does not support Windows Vista x86 and the driver for Windows XP x64 does not contain WHQL certification signature.

For hardware version C1, the drivers for all Windows Oss contain WHQL and DTM certification signature.

During this test, we performed a full function test on Windows Vista driver , Multi-language and Windows 98/ME/2K/XP/XP64 for UI. The Vista driver does not have User Interface, so we used Vista build in tool -Windows Zero configuration for this testing.  The test was started at 02/2 and 1 working days were used to finish it.

Test OS: Windows Vista RTM (Build: 6000).

Test Notebook: Asus W3000 W3H740DD

The left bugs are shown as following:

- Bug ID #2:Installshiled cause Intel graphic driver crash in Greek OS(Fatal Error)

=>LAB: Installshiled cause Intel graphic driver crash in Greek OS.  Update Intel driver could fix this issue.

- Bug ID #8:DWL-G122 C1 install will hanging when using USB1.1 mode (Function Error)

Fail in:  Intel82801BA/BAM USB1.1

Intel82801FB/FBM USB1.1

Intel82801DB/DBM USB1.1

Via Rev5 USB1.1.

=>RD said: It's driver issue. change next version DWL-G122 driver would be ok.

Table of Contents

1. Introduction of Test Product (Present by Product Manager)

2. Basic Function Test	15

2-1 Wireless Band	15

2-2 SSID	15

2-3 SSID Broadcast	16

2-4	Channel	16

2-5	Protocol	16

2-6 Dynamic Rate shifting	16

2-7 Fragment Length	17

2-8 RTS Length	17

2-9 Power Saving	17

2-10 Roaming	17

2-11 DHCP	17

2-12 Login in Windows domain controller	18

2-14 OS Standby Test	18

2-15 Card Hot Swap Test	18

2-16 Windows Zero Configuration Utility-Option	18

2-17 Driver Modifying Protection with a Normal User	19

2-18 D-Link Utility Option (5OS)	19

2-19 PC/Note Book Compatibility	22

2-19-1 PC Compatibility	22

2-19-2 Note Book Compatibility	23

3. Security Test	24

3-1 WEP	24

3-2 WPA-EAP	25

3-3 WPA2-EAP	26

3-4 WPA-PSK	27

3-5 WPA2-PSK	27

3-6 802.1X	27

4. Throughput and Compatibility Test	28

4-1 Card Compatibility Test vs. Other Vendors. (Ad-hoc Mode)	28

4-1-1 802.11b (WinX64)	28

4-1-2 802.11b (Win XP SP2)	28

4-2 AP Compatibility Test vs. Other Vendors (Infrastructure Mode)	29

4-2-1 802.11g (WinX64)	29

4-2-2 802.11g (WinXP SP2)	29

4-2-3 802.11g (Win Vista)	30

4-3 Channel (Ad-Hoc Mode)	31

4-3-1 (WinX64)	31

4-3-2 (Win XP SP2)	31

4-4 Channel (Infrastructure Mode)	32

4-4-1 802.11g (WinX64)	32

4-4-2 802.11b (WinX64)	32

4-4-3 802.11g (WinXP SP2)	33

4-4-4 802.11b (WinXP SP2)	33

5. Endurance Test	34

5-1 Basic Function Endurance	34

5-1-1 Endurance Test (FTP Test in Infrastructure mode)	34

5-1-2 Endurance Test (FTP Test in Ad-Hoc mode)	34

5-2 Security Endurance	34

5-2-1 Endurance Test (Infrastructure mode with Encryption)	34

5-2-2 Endurance Test (Ad-hoc mode with Encryption)	35

6. Other Related Test	36

6-1 Multi-Language (15ML)	36

# 2. Basic Function Test

5OS Test Environment

Server: [PC] ASUS P5SD2-X, ASUS CUV4X, ASUS A8N-SLI, TOSHIBA Tecra A3

[NIC] Intel® PRO/100 VE Network, Intel PRO 21140 10/100, NVIDIA nForce, Intel PRO 100/1000

[OS] Win98 SE,ME, W2K SP4, Win XP SP2, Win XP x64

AP: DWL-3200 / DWL-7700

Firmware: v220.r468.t555eu / v305.dfsus

File Size: 600MB

Vista Test Environment

Server: [PC] GA-8IK1100GT

[NIC] Intel® PRO/100 VE Network

[OS] W2K SP4

AP: DWL-8200

Firmware: r0502 t528-11 jp

File Size: 600MB

## 2-1 Wireless Band

| Description | Result (Pass/ Fail) |
| --- | --- |
| 802.11b (It can support 802.11b only) | Pass |
| 802.11g (It can support 802.11b only and 802.11b&g) | Pass |

## 2-2 SSID

| Description | Result (Pass/ Fail) |
| --- | --- |
| It supports 32 characters | Pass |
| STA SSID:12345678901234567890123456789011 AP SSID:12345678901234567890123456789012 | Pass |
| STA SSID:~!@#$%^&*() AP SSID:~!@#$%^&*() | Pass |
| STA SSID:NULL AP SSID:~!@#$%^&*() | Pass |
| STA SSID:123 45 AP SSID:12345 | Pass |
| “Blank” character | Pass |

## 2-3 SSID Broadcast

| Description | Result (Pass/ Fail) |
| --- | --- |
| Enable | Pass |
| Disable | Pass |

## Channel

| Channel | 802.11b | 802.11g | Ad-Hoc |
| --- | --- | --- | --- |
|  | Result (Pass/Fail) | Result (Pass/Fail) | Result (Pass/Fail) |
| 1 | Pass | Pass | Pass |
| 2 | Pass | Pass | Pass |
| 3 | Pass | Pass | Pass |
| 4 | Pass | Pass | Pass |
| 5 | Pass | Pass | Pass |
| 6 | Pass | Pass | Pass |
| 7 | Pass | Pass | Pass |
| 8 | Pass | Pass | Pass |
| 9 | Pass | Pass | Pass |
| 10 | Pass | Pass | Pass |
| 11 | Pass | Pass | Pass |
| 12 | Pass | Pass | Pass |
| 13 | Pass | Pass | Pass |

## Protocol

| OS Protocol | Win98 SE | Win ME | Win 2000 SP4 | Win XP SP2 | Win XPx64 | VistaX86 |
| --- | --- | --- | --- | --- | --- | --- |
| TCP/IP | Pass | Pass | Pass | Pass | Pass | Pass |
| NetBEUI | Pass | Pass | Pass | Pass | N/A | N/A |
| IPX+ | Pass | Pass | Pass | Pass | Pass | N/A |

- Windows Vista does not support IPX and NETBEUI protocol.

## 2-6 Dynamic Rate shifting

| Description | Result (Pass/ Fail) |
| --- | --- |
| Auto | Pass |

## 2-7 Fragment Length

| Description | Result (Pass/ Fail) |
| --- | --- |
| Default-2346 | Pass |
| 512 | Pass |
| 256 | Pass |

## 2-8 RTS Length

| Description | Result (Pass/ Fail) |
| --- | --- |
| Default-2347 | Pass |
| 512 | Pass |
| 256 | Pass |

## 2-9 Power Saving

| Description | Result (Pass/ Fail) |
| --- | --- |
| Disable | Pass |
| Max Save | Pass |
| Fast Save | Pass |

## 2-10 Roaming

| Description | Result (Pass/ Fail) |
| --- | --- |
| Test a station’s ability to roam between two access points | Pass |

## 2-11 DHCP

| Description | Result (Pass/ Fail) |
| --- | --- |
| Card can get IP from the DHCP server through an AP | Pass |

## 2-12 Login in Windows domain controller

Server: Windows 2000 advance server as a Domain Controller

| OS | Security | Result (Pass/Fail) |
| --- | --- | --- |
| Win 98 SE | 802.1X + EAP-TTLS with MSCHAPV2 | Pass |
| Win 2000 SP4 | Open System with Non-Encryption | Pass |
| Win XP SP2 | Open System + WEP 128 bits Hex Key | Pass |
| Win XP x64 | 802.1X + EAP-TTLS with MSCHAPV2 | Pass |
| Win Vistax86 | N/A | N/A |

- Windows 2000 Server Login is not supported by Windows Vista.

## 2-14 OS Standby Test

| OS | Result (Pass/Fail) |
| --- | --- |
| WinXP64 SP1 | Pass |
| Win XP Pro. SP2 | Pass |
| Win2000 Pro. SP4 | Pass |
| Win ME | Pass |
| Win98 SE | Pass |
| Win Vistax86 | Pass |

## 2-15 Card Hot Swap Test

| OS Protocol | Win98 SE | Win ME | Win2000 SP4 | Win XP SP2 | Win XP x64 | Win Vistax86 |
| --- | --- | --- | --- | --- | --- | --- |
| Disable/Enable adapter | Pass | Pass | Pass | Pass | Pass | Pass |
| Plug in/Pull out the adapter | Pass | Pass | Pass | Pass | Pass | Pass |

## 2-16 Windows Zero Configuration Utility-Option

| Description | Result (Pass/Fail) |
| --- | --- |
| Infrastructure | Pass |
| Ad-Hoc | Pass |

## 2-17 Driver Modifying Protection with a Normal User

| Description | Result (Pass/Fail) |
| --- | --- |
| Win XP x64 Edition | Pass |
| Win XP SP2 | Pass |
| Win 2000 SP4 | Pass |

## 2-18 D-Link Utility Option (5OS)

| Test Item | Default | Test Result | DUT version |
| --- | --- | --- | --- |
| Link Info |  |  |  |
| Status | No connection to Network | Pass |  |
| SSID |  | Pass |  |
| Frequency |  | Pass |  |
| Wireless Mode |  | Pass |  |
| Encryption |  | Pass |  |
| Connection Info |  | Pass |  |
| Tx Rate |  | Pass |  |
| Channel |  | Pass |  |
| Rescan |  | Pass |  |
| Signal Strength |  | Pass |  |
| Transmit Packet Count |  | Pass |  |
| Receive Packet Count |  | Pass |  |
| Configuration |  |  |  |
| SSID |  | Pass |  |
| Wireless Mode | Infrastructure | Pass |  |
| Authentication |  | Pass |  |
| Data Encryption |  | Pass |  |
| Key Length |  | Pass |  |
| IEEE 802.1x |  | Pass |  |
| Authentication Config |  | Pass |  |
| Advanced |  |  |  |
| WPA Pass Phrase |  | Pass |  |
| EAP_TYPE | EAP_TLS | Pass |  |
| User Certificate |  | Pass |  |
| Validate Server Certificate |  | Pass |  |
| User Name |  | Pass |  |
| Domain Name |  | Pass |  |
| Password |  | Pass |  |
| Confirm password |  | Pass |  |
| TTLS Identity |  | Pass |  |
| Trusted Server Configuration (Add and Remove) |  | Pass |  |
| IP Setting (In button “IP Settings”) |  |  |  |
| Obtain an IP Address Automatically |  | Pass |  |
| Assign the following IP Address |  | Pass |  |
| IP Address |  | Pass |  |
| Sub Mask |  | Pass |  |
| Default Gateway |  | Pass |  |
| Obtain DNS Server Address Automatically |  | Pass |  |
| Use the following DNS Server Address |  | Pass |  |
| Preferred DNS Server |  | Pass |  |
| Alternate DNS Server |  | Pass |  |
| Apply with Profile Proxy |  | Pass |  |
| Proxy Setting (In “IP Settings” -> “Proxy”) |  |  |  |
| Automatically detect settings |  | Pass |  |
| Use automatic configuration |  | Pass |  |
| Automatic configuration server address |  | Pass |  |
| Use a proxy server |  | Pass |  |
| By pass proxy server for local address |  | Pass |  |
| Use the same proxy server for all protocols |  | Pass |  |
| HTTP |  | Pass |  |
| Secure |  | Pass |  |
| FTP |  | Pass |  |
| Gopher |  | Pass |  |
| Socks |  | Pass |  |
| Exceptions |  | Pass |  |
| Advanced |  |  |  |
| Frequency (Default: Auto) |  | Pass |  |
| Ad-Hoc Channel (Default: Channel 6) |  | Pass |  |
| Profile IP Settings (Default: Disable) |  | Pass |  |
| Power Mode (Default: Disable) |  | Pass |  |
| Launch Utility on Startup (Default: Enable) |  | Pass |  |
| Fragmentation Threshold | 2346 | Pass |  |
| RTS Threshold | 2347 | Pass |  |
| Site Survey |  |  |  |
| Available Network |  | Pass |  |
| Profile (Button “↑” and “↓”) |  | Pass |  |
| Refresh |  | Pass |  |
| Configure |  | Pass |  |
| Advanced |  | Pass |  |
| Add |  | Pass |  |
| Remove |  | Pass |  |
| Properties |  | Pass |  |
| Connect |  | Pass |  |
| Rescan |  | Pass |  |

## 2-19 PC/Note Book Compatibility

### 2-19-1 PC Compatibility

Driver Team Motherboard List

| No. | Ser No. | Model Name | CPU | BIOS | MB Chipset |  |  | BUS |  | USB Chip |  | OS |  | Test Conclusion (Pass/Fail) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 75 | TLD2075D | ASUS P4P800 | P4 2.8 | AMI |  | Intel 865PE | PCI |  | PCI 2.2, PCI 2.3, USB 2.0 |  | Win XP SP2 |  | Put 600MB |  | Pass |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Get 600MB |  | Pass |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | S1 Standby |  | Pass |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | S4 Hibernate |  | Pass |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Enable/Disable |  | Pass |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Hot swap |  | Pass |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Uninstall |  | Pass |  |
|  |  | Asus P5SD2-X | P4 3.0G HT/EM64T | AMI BIOS |  | Intel 915p Intel ICH6 | PCI |  | USB2.0 |  | Win 2000 SP4 |  | Put 600MB |  | Pass |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Get 600MB |  | Pass |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | S1 Standby |  | Pass |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | S4 Hibernate |  | Pass |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Enable/Disable |  | Pass |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Hot swap |  | Pass |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Uninstall |  | Pass |  |
|  |  | ASUS A8N-SLI | AMD Athlon 64 3200+ | AMI BIOS |  | NVIDIA nForce 4 Ultra | PCI |  | USB2.0 |  | Win XP SP2 |  | Put 600MB |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Get 600MB |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | S1 Standby |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | S4 Hibernate |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Enable/Disable |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Hot swap |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Uninstall |  |  | Pass |
|  |  | Asus CUV4X | Intel Pentium III 800 | Award |  | Intel 915p Intel ICH6 | PCI |  | USB2.0 |  | Win 98 SE |  | Put 600MB |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Get 600MB |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | S1 Standby |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | S4 Hibernate |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Enable/Disable |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Hot swap |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Uninstall |  |  | Pass |
|  |  | Asus P5B-VM | Intel Core 2 6300 | AMI BIOS |  | Intel G965 | PCI |  | USB2.0 |  | Win XP x64 |  | Put 600MB |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Get 600MB |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | S1 Standby |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | S4 Hibernate |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Enable/Disable |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Hot swap |  |  | Pass |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Uninstall |  |  | Pass |

### 2-19-2 Note Book Compatibility

| No. | Product name | Notebook Model | Processor type | Processor Speed | MB Chipset | USB Chip | OS | Test Conclusion (Pass/Fail) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TLD2N021 | COMPAQ | Presario X1000 | Intel Pentium M | 1.3 GHz |  | Intel® 82801DB/DBM USB2.0 | Windows XP SP2 | Put 600MB | Pass |
|  |  |  |  |  |  |  |  | Get 600MB | Pass |
|  |  |  |  |  |  |  |  | S1 Standby | Pass |
|  |  |  |  |  |  |  |  | S4 Hibernate | Pass |
|  |  |  |  |  |  |  |  | Enable/Disable | Pass |
|  |  |  |  |  |  |  |  | Hot swap | Pass |
|  |  |  |  |  |  |  |  | Uninstall | Pass |
|  | TOSHIBA | Tecra A3 | Intel Pentium M | 1.6 GHz | Intel 945M | Intel 82801 Family | Windows XP SP2 | Put 600MB | Pass |
|  |  |  |  |  |  |  |  | Get 600MB | Pass |
|  |  |  |  |  |  |  |  | S1 Standby | Pass |
|  |  |  |  |  |  |  |  | S4 Hibernate | Pass |
|  |  |  |  |  |  |  |  | Enable/Disable | Pass |
|  |  |  |  |  |  |  |  | Hot swap | Pass |
|  |  |  |  |  |  |  |  | Uninstall | Pass |

# 3. Security Test

5OS Test Environment

Server:

[PC] GA-7N400L

[NIC] Realtek

[OS] W2K + SP4

AP: DWL-3200 & Cisco 1200

Tools: Airopeek

Vista Test Environment

Server:

[PC] GA-8I1100GT

[NIC] Inetl® PRO/100 VE Network Connection

[OS] W2K + SP4

AP: DWL-8200 & Cisco 1200 & DI-624 C2

Tools: Airopeek

## 3-1 WEP

| Item | Authentication | Key type | Key Size | Valid Key | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| 1 | Open System | Hex | 64 Bits | First | Pass |
| 2 | Open System | Hex | 64 Bits | Second | Pass |
| 3 | Open System | Hex | 64 Bits | Third | Pass |
| 4 | Open System | Hex | 64 Bits | Fourth | Pass |
| 5 | Open System | Hex | 128 Bits | First | Pass |
| 6 | Open System | Hex | 128 Bits | Second | Pass |
| 7 | Open System | Hex | 128 Bits | Third | Pass |
| 8 | Open System | Hex | 128 Bits | Fourth | Pass |
| 9 | Open System | ASCII | 64 Bits | First | Pass |
| 10 | Open System | ASCII | 64 Bits | Second | Pass |
| 11 | Open System | ASCII | 64 Bits | Third | Pass |
| 12 | Open System | ASCII | 64 Bits | Fourth | Pass |
| 13 | Open System | ASCII | 128 Bits | First | Pass |
| 14 | Open System | ASCII | 128 Bits | Second | Pass |
| 15 | Open System | ASCII | 128 Bits | Third | Pass |
| 16 | Open System | ASCII | 128 Bits | Fourth | Pass |
| 17 | Shared Key | Hex | 64 Bits | First | Pass |
| 18 | Shared Key | Hex | 64 Bits | Second | Pass |
| 19 | Shared Key | Hex | 64 Bits | Third | Pass |
| 20 | Shared Key | Hex | 64 Bits | Fourth | Pass |
| 21 | Shared Key | Hex | 128 Bits | First | Pass |
| 22 | Shared Key | Hex | 128 Bits | Second | Pass |
| 23 | Shared Key | Hex | 128 Bits | Third | Pass |
| 24 | Shared Key | Hex | 128 Bits | Fourth | Pass |
| 25 | Shared Key | ASCII | 64 Bits | First | Pass |
| 26 | Shared Key | ASCII | 64 Bits | Second | Pass |
| 27 | Shared Key | ASCII | 64 Bits | Third | Pass |
| 28 | Shared Key | ASCII | 64 Bits | Fourth | Pass |
| 29 | Shared Key | ASCII | 128 Bits | First | Pass |
| 30 | Shared Key | ASCII | 128 Bits | Second | Pass |
| 31 | Shared Key | ASCII | 128 Bits | Third | Pass |
| 32 | Shared Key | ASCII | 128 Bits | Fourth | Pass |
| 33 | Open/Shared | Hex | 64 Bits | First | Pass |
| 34 | Open/Shared | Hex | 64 Bits | Second | Pass |
| 35 | Open/Shared | Hex | 64 Bits | Third | Pass |
| 36 | Open/Shared | Hex | 64 Bits | Fourth | Pass |
| 37 | Open/Shared | Hex | 128 Bits | First | Pass |
| 38 | Open/Shared | Hex | 128 Bits | Second | Pass |
| 39 | Open/Shared | Hex | 128 Bits | Third | Pass |
| 40 | Open/Shared | Hex | 128 Bits | Fourth | Pass |
| 41 | Open/Shared | ASCII | 64 Bits | First | Pass |
| 42 | Open/Shared | ASCII | 64 Bits | Second | Pass |
| 43 | Open/Shared | ASCII | 64 Bits | Third | Pass |
| 44 | Open/Shared | ASCII | 64 Bits | Fourth | Pass |
| 45 | Open/Shared | ASCII | 128 Bits | First | Pass |
| 46 | Open/Shared | ASCII | 128 Bits | Second | Pass |
| 47 | Open/Shared | ASCII | 128 Bits | Third | Pass |
| 48 | Open/Shared | ASCII | 128 Bits | Fourth | Pass |

## 3-2 WPA-EAP

| Item | Cipher Type | EAP | Radius | 5OS Result (Pass/Fail) | Vista Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| 1 | TKIP | EAP-TLS | IAS | Pass | Pass |
| 2 | TKIP | PEAP | IAS | Pass | Pass |
| 3 | TKIP | EAP-TLS | Odyssey | Pass | Pass |
| 4 | TKIP | LEAP | Odyssey | Pass | N/A |
| 5 | TKIP | EAP-TTLS(PAP) | Odyssey | Pass | N/A |
| 6 | TKIP | EAP-TTLS(MSCHAP) | Odyssey | Pass | N/A |
| 7 | TKIP | EAP-TTLS(MSCHAPV2) | Odyssey | Pass | N/A |
| 8 | TKIP | PEAP(EAP-MSCHAPV2) | Odyssey | Pass | Pass |
| 9 | AES | EAP-TLS | IAS | Pass | Pass |
| 10 | AES | PEAP | IAS | Pass | Pass |
| 11 | AES | EAP-TLS | Odyssey | Pass | Pass |
| 12 | AES | EAP-TTLS(PAP) | Odyssey | Pass | N/A |
| 13 | AES | EAP-TTLS(MSCHAP) | Odyssey | Pass | N/A |
| 14 | AES | EAP-TTLS(MSCHAPV2) | Odyssey | Pass | N/A |
| 15 | AES | PEAP(EAP-MSCHAPV2) | Odyssey | Pass | N/A |
| 16 | Auto | EAP-TLS | IAS | Pass | Pass |
| 17 | Auto | PEAP | IAS | Pass | Pass |
| 18 | Auto | EAP-TLS | Odyssey | Pass | Pass |
| 19 | Auto | EAP-TTLS(PAP) | Odyssey | Pass | N/A |
| 20 | Auto | EAP-TTLS(MSCHAP) | Odyssey | Pass | N/A |
| 21 | Auto | EAP-TTLS(MSCHAPV2) | Odyssey | Pass | N/A |
| 22 | Auto | PEAP(EAP-MSCHAPV2) | Odyssey | Pass | Pass |

- Now the driver for Windows Vista x86 uses Windows’ supplicant to create WPA-EAP, WPA2-EAP and 802.1x connections.  Because Windows Vista does not provide APIs to support vendors to call their supplicant and fill in connection parameters, therefore the setting parameters in UI are unavailable in Windows Vista x86 and users need to re-input their connection parameters with the pop-up dialog box from Windows Vista.

## 3-3 WPA2-EAP

| Item | Cipher Type | EAP | Radius | 5OS Result (Pass/Fail) | Vista Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| 1 | TKIP | EAP-TLS | IAS | Pass | Pass |
| 2 | TKIP | PEAP | IAS | Pass | Pass |
| 3 | TKIP | EAP-TLS | Odyssey | Pass | Pass |
| 4 | TKIP | LEAP | Odyssey | Pass | N/A |
| 5 | TKIP | EAP-TTLS(PAP) | Odyssey | Pass | N/A |
| 6 | TKIP | EAP-TTLS(MSCHAP) | Odyssey | Pass | N/A |
| 7 | TKIP | EAP-TTLS(MSCHAPV2) | Odyssey | Pass | N/A |
| 8 | TKIP | PEAP(EAP-MSCHAPV2) | Odyssey | Pass | Pass |
| 9 | AES | EAP-TLS | IAS | Pass | Pass |
| 10 | AES | PEAP | IAS | Pass | Pass |
| 11 | AES | EAP-TLS | Odyssey | Pass | Pass |
| 12 | AES | EAP-TTLS(PAP) | Odyssey | Pass | N/A |
| 13 | AES | EAP-TTLS(MSCHAP) | Odyssey | Pass | N/A |
| 14 | AES | EAP-TTLS(MSCHAPV2) | Odyssey | Pass | N/A |
| 15 | AES | PEAP(EAP-MSCHAPV2) | Odyssey | Pass | N/A |
| 16 | Auto | EAP-TLS | IAS | Pass | Pass |
| 17 | Auto | PEAP | IAS | Pass | Pass |
| 18 | Auto | EAP-TLS | Odyssey | Pass | Pass |
| 19 | Auto | EAP-TTLS(PAP) | Odyssey | Pass | N/A |
| 20 | Auto | EAP-TTLS(MSCHAP) | Odyssey | Pass | N/A |
| 21 | Auto | EAP-TTLS(MSCHAPV2) | Odyssey | Pass | N/A |
| 22 | Auto | PEAP(EAP-MSCHAPV2) | Odyssey | Pass | Pass |

- Now the driver for Windows Vista x86 uses Windows’ supplicant to create WPA-EAP, WPA2-EAP and 802.1x connections.  Because Windows Vista does not provide APIs to support vendors to call their supplicant and fill in connection parameters, therefore the setting parameters in UI are unavailable in Windows Vista x86 and users need to re-input their connection parameters with the pop-up dialog box from Windows Vista.

- WPA2 mode is not supported under Win 98SE and Win ME environment.

## 3-4 WPA-PSK

| Item | Cipher Type | Result (Pass/Fail) |
| --- | --- | --- |
| 1 | TKIP | Pass |
| 2 | AES | Pass |
| 3 | It supports 8~64 bits | Pass |

## 3-5 WPA2-PSK

| Item | Cipher Type | Result (Pass/Fail) |
| --- | --- | --- |
| 1 | TKIP | Pass |
| 2 | AES | Pass |
| 3 | It supports 8~64 bits | Pass |

## 3-6 802.1X

| Item | Authentication Mode | Authentication Server | Data Encryption type | EAP-Type | 5OS Result (Pass/Fail) | Vista Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Open | Odyssey server 2.0 | WEP | EAP-TLS | Pass | Pass |
| 2 | Open | Odyssey server 2.0 | WEP | LEAP | Pass | Pass |
| 3 | Open | Odyssey server 2.0 | WEP | EAP-TTLS(PAP) | Pass | Pass |
| 4 | Open | Odyssey server 2.0 | WEP | EAP-TTLS(MSCHAP) | Pass | N/A |
| 5 | Open | Odyssey server 2.0 | WEP | EAP-TTLS(MSCHAPV2) | Pass | N/A |
| 6 | Open | Odyssey server 2.0 | WEP | PEAP (EAP-MSCHAP V2) | Pass | N/A |
| 7 | Open | IAS | WEP | PEAP (EAP-MSCHAPv2) | Pass | N/A |
| 8 | Open | IAS | WEP | EAP-TLS | Pass | Pass |

- Now the driver for Windows Vista x86 uses Windows’ supplicant to create WPA-EAP, WPA2-EAP and 802.1x connections.  Because Windows Vista does not provide APIs to support vendors to call their supplicant and fill in connection parameters, therefore the setting parameters in UI are unavailable in Windows Vista x86 and users need to re-input their connection parameters with the pop-up dialog box from Windows Vista.

# 4. Throughput and Compatibility Test

## 4-1 Card Compatibility Test vs. Other Vendors. (Ad-hoc Mode)

Test Environment

Server: Toshiba A3

OS [Win XP SP2]

Station: ASUS P5P800-VM

OS [Win XP SP2]

Test tool: Chariot 5.0

Channel: 6

Place: B3

### 4-1-1 802.11b (WinX64)

File Size: 1MB

| Item | Chariot Unit: Mbps |  |  |  |
| --- | --- | --- | --- | --- |
|  | No WEP |  | WEP-128bits |  |
|  | TX | RX | TX | RX |
| ##### Buffalo CB54G | 5.469 | 5.953 | 5.436 | 5.925 |
| ###### Cisco CB21AG | 5.246 | 5.203 | 5.204 | 5.176 |
| Proxim-600 | 5.204 | 5.074 | 5.173 | 5.054 |
| TI  G122 C1 | 5.316 | 5.907 | 5.154 | 5.826 |
| TI  Ralink公版 | 5.116 | 5.881 | 5.107 | 5.744 |
| DWL-G122 C1 | 5.309 | 5.458 | 5.287 | 5.484 |

### 4-1-2 802.11b (Win XP SP2)

File Size: 1MB

| Item | Chariot Unit: Mbps |  |  |  |
| --- | --- | --- | --- | --- |
|  | No WEP |  | WEP-128bits |  |
|  | TX | RX | TX | RX |
| ##### Buffalo CB54G | 5.413 | 5.928 | 5.399 | 5.905 |
| ###### Cisco CB21AG | 5.252 | 5.176 | 5.212 | 5.165 |
| Proxim-600 | 5.208 | 5.113 | 5.211 | 5.186 |
| TI | 5.345 | 5.001 | 5.116 | 5.213 |
| DWL-G122 C1 | 5.484 | 5.801 | 5.279 | 5.746 |

## 4-2 AP Compatibility Test vs. Other Vendors (Infrastructure Mode)

Test Environment

Server: Compaq X1000

OS [Win XP SP2]

Station: ASUS P5P800-VM

OS [Win XP SP2]

Test tool: Chariot 5.0

Channel:  802.11b/g  6

802.11a  52

### 4-2-1 802.11g (WinX64)

File Size: 1MB

| Item | F/W | Chariot Unit: Mbps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | No WEP |  | WEP-128bits |  | WPA-PSK |  |
|  |  | TX | RX | TX | RX | TX | RX |
| DI-624 D2 (Cameo) | 4.03 | 22.118 | 23.070 | 22.868 | 21.631 | 21.676 | 22.318 |
| DWL-8200 | K9W7-M | 22.411 | 23.354 | 22.201 | 23.043 | 21.842 | 22.700 |
| DI-624 C3 (Cameo) | 2.76 | 21.916 | 22.589 | 21.614 | 22.358 | 21.709 | 22.118 |
| DI-784 | 2.40 | 22.220 | 24.872 | 21.259 | 24.354 | 20.617 | 23.754 |

### 4-2-2 802.11g (WinXP SP2)

File Size: 1MB

| Item | F/W | Chariot Unit: Mbps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | No WEP |  | WEP-128bits |  | WPA-PSK |  |
|  |  | TX | RX | TX | RX | TX | RX |
| DI-624 D2 (Cameo) | 4.03 | 22.075 | 22.542 | 21.825 | 22.448 | 21.428 | 22.144 |
| DWL-8200 | K9W7-M | 22.331 | 23.091 | 22.023 | 22.895 | 21.725 | 21.482 |
| DI-624 C3 (Cameo) | 2.76 | 22.072 | 21.115 | 21.653 | 22.578 | 21.693 | 22.613 |
| DI-784 | 2.40 | 19.205 | 22.579 | 22.152 | 24.766 | 22.122 | 24.049 |

### 4-2-3 802.11g (Win Vista)

Test Environment

Server:

[PC] GA-8I1100GT

[NIC] Inetl® PRO/100 VE Network Connection

[OS] W2K + SP4

Tools: Airopeek

CH:11

| Item | F/W | Chariot Unit: Mbps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | WPA2-PSK-AES |  | WEP-128bits |  | WPA-PSK-TKIP |  |
|  |  | TX | RX | TX | RX | TX | RX |
| Cisco AP1231G-A-K9 | 12.3(8)JEA | 22.065 | 20.503 | 21.868 | 21.631 | 21.676 | 21.358 |
| Broadcom | 3.131.35.0 | 22.411 | 21.354 | 21.201 | 21.043 | 21.842 | 21.740 |
| Atheros AR5002AP-2X | 0.0.9.97 | 21.916 | 21.589 | 21.614 | 21.328 | 21.709 | 21.138 |
| Conexant ISL39300A | 2.6.11.0 | 21.075 | 21.872 | 21.259 | 21.324 | 20.617 | 21.724 |

## 4-3 Channel (Ad-Hoc Mode)

### 4-3-1 (WinX64)

Test Environment

Server: Compaq X1000 [OS: Win XP SP2]

Station: ASUS P5P800-VM [OS: Win XP SP2]

Test tool: Chariot 5.0

File Size: 1MB

| Vendors Card | Channel | Chariot Unit: Mbps |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | No WEP |  | WEP-128bits |  |
|  |  | TX | RX | TX | RX |
| ##### Buffalo CB54G | 1 | 5.159 | 5.729 | 5.106 | 5.719 |
|  | 6 | 5.469 | 5.953 | 5.436 | 5.925 |
|  | 11 | 5.447 | 5.937 | 5.384 | 5.849 |
| ###### Cisco CB21AG  G122 C1 | 1 | 5.048 | 5.422 | 5.887 | 5.179 |
|  | 6 | 5.246 | 5.203 | 5.204 | 5.176 |
|  | 11 | 5.248 | 5.211 | 5.223 | 5.188 |
| ###### Cisco CB21AG ######  Ralink公版 | 1 | 5.174 | 5.114 | 5.151 | 5.118 |
| ###### DWL-G122 C1  G122 C1 | 1 | 5.322 | 5.482 | 5.729 | 5.062 |
|  | 6 | 5.309 | 5.458 | 5.287 | 5.484 |
|  | 11 | 5.299 | 5.498 | 5.291 | 5.463 |

### 4-3-2 (Win XP SP2)

File Size: 1MB

| Vendors Card | Channel | Chariot Unit: Mbps |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | No WEP |  | WEP-128bits |  |
|  |  | TX | RX | TX | RX |
| ##### Buffalo CB54G | 1 | 5.923 | 5.385 | 5.961 | 5.347 |
|  | 6 | 5.413 | 5.928 | 5.399 | 5.905 |
|  | 11 | 5.484 | 5.973 | 5.450 | 5.749 |
| ###### Cisco CB21AG  G122 C1 | 1 | 5.032 | 5.599 | 5.010 | 5.431 |
|  | 6 | 5.252 | 5.176 | 5.212 | 5.165 |
|  | 11 | 5.264 | 5.259 | 5.361 | 5.385 |
| ###### DWL-G122 C1  G122 C1 | 1 | 5.312 | 5.459 | 5.292 | 5.475 |
|  | 6 | 5.484 | 5.801 | 5.279 | 5.746 |
|  | 11 | 5.481 | 5.792 | 5.292 | 5.474 |

## 4-4 Channel (Infrastructure Mode)

Test Environment

Server: Compaq X1000

OS [Win XP SP2]

Station: ASUS P5P800-VM

OS [Win XP SP2]

Test tool: Chariot 5.0

### 4-4-1 802.11g (WinX64)

File Size: 1MB

| AP | Version | Channel | Chariot Unit: Mbps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | No WEP |  | WEP-128bits |  | WPA-PSK |  |
|  |  |  | TX | RX | TX | RX | TX | RX |
| DI-624 D2 (Cameo) G122 | 4.03 | 1 | 20.686 | 20.345 | 20.574 | 20.112 | 20.275 | 20.365 |
|  |  | 6 | 22.118 | 23.070 | 22.868 | 21.631 | 21.676 | 22.318 |
|  |  | 11 | 21.109 | 22.356 | 20.116 | 21.219 | 20.030 | 22.152 |
| DI-624 D2 (Cameo)  公版Ralink | 4.03 | 1 | 21.436 | 21.546 | 21.670 | 22.058 | 20.799 | 20.965 |
| D-Link DWL-8200 AP  G122 | 1.20 | 1 | 19.166 | 20.110 | 19.098 | 20.308 | 19.832 | 20.054 |
|  |  | 6 | 22.411 | 23.354 | 22.201 | 23.043 | 21.842 | 22.700 |
|  |  | 11 | 22.467 | 23.381 | 22.128 | 23.099 | 21.589 | 22.777 |
| D-Link DWL-8200 AP  公版Ralink | 1.20 | 1 | 20.613 | 20.470 | 20.818 | 20.589 | 20.425 | 20.174 |

### 4-4-2 802.11b (WinX64)

File Size: 1MB

| AP | Version | Channel | Chariot Unit: Mbps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | No WEP |  | WEP-128bits |  | WPA-PSK |  |
|  |  |  | TX | RX | TX | RX | TX | RX |
| D-Link DWL-8200 AP  G122 C1 | 1.20 | 1 | 5.868 | 5.454 | 5.789 | 5.435 | 5.831 | 5.522 |
|  |  | 6 | 5.168 | 5.214 | 5.150 | 5.176 | 5.095 | 5.150 |
|  |  | 11 | 5.166 | 5.230 | 5.138 | 5.213 | 5.089 | 5.155 |
| D-Link DWL-8200 AP Ralink公版 | 1.20 | 1 | 5.585 | 5.974 | 5.565 | 5.836 | 5.536 | 5.906 |

### 4-4-3 802.11g (WinXP SP2)

File Size: 1MB

| AP | Version | Channel | Chariot Unit: Mbps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | No WEP |  | WEP-128bits |  | WPA-PSK |  |
|  |  |  | TX | RX | TX | RX | TX | RX |
| DI-624 C3 (Cameo) | 2.76 | 1 | 22.143 | 23.083 | 21.834 | 22.369 | 21.641 | 22.274 |
|  |  | 6 | 22.072 | 21.115 | 21.653 | 22.578 | 21.693 | 22.613 |
|  |  | 11 | 22.051 | 22.862 | 21.910 | 22.916 | 21.715 | 22.743 |
| DI-624 D2 (Cameo) | 4.03 | 1 | 22.225 | 23.108 | 21.846 | 22.843 | 21.701 | 22.698 |
|  |  | 6 | 22.075 | 22.542 | 21.825 | 22.448 | 21.428 | 22.144 |
|  |  | 11 | 22.161 | 23.106 | 21.915 | 22.935 | 21.746 | 22.723 |

### 4-4-4 802.11b (WinXP SP2)

File Size: 1MB

| AP | Version | Channel | Chariot Unit: Mbps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | No WEP |  | WEP-128bits |  | WPA-PSK |  |
|  |  |  | TX | RX | TX | RX | TX | RX |
| D-Link DWL-8200 AP | 1.20 | 1 | 5.077 | 5.166 | 5.059 | 5.145 | 5.070 | 5.088 |
|  |  | 6 | 5.155 | 5.203 | 5.145 | 5.218 | 5.085 | 5.154 |
|  |  | 11 | 5.163 | 5.230 | 5.142 | 5.201 | 5.103 | 5.168 |

# 5. Endurance Test

## 5-1 Basic Function Endurance

Test Environment

File Size: 600MB

Test tool: Chariot 5.0
Run: Over 12 hrs

### 5-1-1 Endurance Test (FTP Test in Infrastructure mode)

| Item | PC/Note Book | OS | AP | Channel | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| STA1 | ASUS P5SD2-X | Windows XP SP2 | DWL-3200 | 6 | Pass |
| STA2 | ASUS P4P800 | Windows 2000 SP4 | DWL-7200 | 11 | Pass |

### 5-1-2 Endurance Test (FTP Test in Ad-Hoc mode)

| Item | PC/Note Book | OS | Channel | Result (Pass/Fail) |
| --- | --- | --- | --- | --- |
| STA1 | ASUS P4P800 | Windows XP SP2 | 6 | Pass |
| STA2 | ASUS P5VMB | Windows XP x64 | 6 | Pass |

## 5-2 Security Endurance

### 5-2-1 Endurance Test (Infrastructure mode with Encryption)

Test Environment

File Size: 600MB

Utility: FTP command
Run: Over 12 hrs

| STA Mode | PC/Note Book | OS | AP | Encryption | Channel | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- | --- |
| STA1 | ASUS CUV4X | Windows 98 SE | DWL-77700 | 802.1X + EAP-TTLS with MSCHAPV2 | 6 | Pass |
| STA2 | ASUS P5B-VM | Windows XP x64 | DWL-7700 | 802.1X + PEAP with EAP-MSCHAPV2 | 11 | Pass |
| STA3 | MSI K8T Neo 2 | Windows Vista | DWL-2100 | WPA2 + EAP-TLS | 11 | Pass |

### 5-2-2 Endurance Test (Ad-hoc mode with Encryption)

Test Environment

File Size: 600MB

Utility: FTP command
Run: Over 12 hrs

| STA Mode | PC/Note Book | OS | Channel | Encryption | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| STA1 | Toshiba Tecra A3 | Windows XP SP2 | 6 | Open + WEP Hex 128 bits key | Pass |
| STA2 | ASUS P5B-VM | Windows XP x64 | 6 | Open + WEP Hex 128 bits key | Pass |
| STA3 | ASUS A8N-SLI | Windows XP x64 | 11 | Open + WEP ASCII 128 bits key | Pass |
| STA4 | ASUS P5SD2-X | Windows 2000 SP4 | 11 | Open + WEP ASCII 128 bits key | Pass |

# 6. Other Related Test

## 6-1 Multi-Language (15ML)

Test environment

Compaq Presario X1000 [OS: Windows XP]

Toshiba Tecra A3 [OS: Windows XP]

ASUS P5P800-VM [OS: Windows XP]

ASUS P4B [OS: Windows XP]

| Language | Install shield | Wireless Connection Wizard |
| --- | --- | --- |
|  | Result (Pass/Fail) | Result (Pass/Fail) |
| English | Pass | Pass |
| Danish | Pass | Pass |
| Dutch | Pass | Pass |
| Czech | Pass | Pass |
| Finnish | Pass | Pass |
| French | Pass | Pass |
| German | Pass | Pass |
| Greek | Fail | Pass |
| Hungarian | Pass | Pass |
| Italian | Pass | Pass |
| Norwegian | Pass | Pass |
| Polish | Pass | Pass |
| Portuguese | Pass | Pass |
| Spanish | Pass | Pass |
| Swedish | Pass | Pass |

- Refer to bug #11