| D-Link DWL-G630 Wireless Card Tested/Verified Report Product Model; DWL-G630 (18ML). Product Description; Card Bus. Driver Version; 1.1.1.0 (5 Windows) 2.0.3.0 (Vista x86). Utility Version; 5.10(5 Windows). Setup Version; S0024. Reg. Region; ETSI. H.W Version; E2. PCB Version; 2A1. Product Key Component; Ralink RT2561+2527. No.8 Li-Shing 7th Rd., Science-based Industrial Park, Hsinchu, Taiwan,R.O.C. TEL:886-3-5636666 |
| --- |
| Approved by | Checked by | Department | Prepared by |
| Margaret Lee(#3626) Margaret_Lee@alphanetworks.com | Margaret Lee(#3626) Margaret_Lee@alphanetworks.com | TLD2 | Daby Chuang(#3613) Daby_Chuang@Alphanetworks.com Jimmy Yeh(#3622) Jimmy_Yeh@Alphanetworks.com |

Plan Revision History

| Rev. | Date | Author | Reason for Changes |
| --- | --- | --- | --- |
| A1 | 2004/09/01 | Daby | First version. |
| A2 | 2004/09/20 | Emily | Update Security section |
| A3 | 2004/09/29 | Andy | Add firewall in Win XP with SP2 function |
| A4 | 2004/11/22 | Daby | Update PC/Note Book list |
| A5 | 2004/12/01 | Daby | Update Channel function |
| A5.1 | 2004/12/07 | Emily | Update Basic function for conceptronic UI |
| A6 | 2005/03/09 | Daby | Add Atheros Utility |
| A7 | 2005/03/10 | Daby | Add WPA2 Security function |
| A8 | 2005/05/11 | Daby | Add WMM (Wi-Fi Multimedia) function |
| A9 | 2005/09/30 | Daby | Add JumpStart function |
| A10 | 2005/11/01 | Daby | Add XP_64Bit function |
| A11 | 2006/01/04 | Daby | Add in Super Mode Endurance test <Item5-3> |
| A12 | 2006/12/04 | Daby | Add in Super Mode to copy big file and run “Rescan” from utility <Item 2-12-1> # 60103SM01 |
| A13 | 2006/01/10 | Daby | Add in NTFS file system in Win XP with SP2 <Item 2-5> # 60105SM01 |
| A14 | 2006/02/21 | Daby | Add in domain login by WPA-TLS and WPA2-TLS |
| A15 | 2006/06/12 | Daby | Add JumpStart function <Item 6-5-1, 6-5-2> |
| A16 | 2006/06/13 | Daby | Add Channel function <2-4-1, 2-4-2, 2-4-3> |

Product Release History

| Date | Release Version | RD | Solved Bug Description |
| --- | --- | --- | --- |
| 2006/01/16 | Utility Version: V5.10 Driver Version: 1.0.3.0 Ralink Build Version: 3.3.1.51123 Setup Version: S0006 | Lily Hsu | - Old H/W and New Driver - Support OS: Windows98SE, Win ME, Win2K(SP4), Win XP(SP2), WinXP64bit 3. Support WMM function |
| 2006/05/15 | Utility Version: v 5.10 Driver Version: 1.01.00.0000 Build Version:3.3.1.60511 Setup Version: S0008 | Lily Hsu | - Ralink update the new driver 1.1.0.0. - Change the H/W to RoHS. |
| 2006/08/01 | Utility Version: v 5.10 (4OS) v1.00 (X64) Driver Version: 1.01.01.0000 Build Version:3.3.1.60728 Setup Version: S0012 | Lily Hsu | - Support WMM. - Support ISAREL channel 1~13. - Support WMM Power Save. - Limit username and password to 64 characters. |
| 2006/08/09 | Utility Version: v 5.10 (4OS) v1.00 (X64) Driver Version: 1.01.01.0000 Build Version:3.3.1.60728 Setup Version: S0013 | Lily Hsu | Fix bug: - Copy Right change to 2006 - Install driver error issue - Setup version release to S0013 |
| 2007/01/24 | Utility: V1.00 (Build122-S0001) Driver: Windows 98SE/ME/2000/XP/XP x64 -- 1.2.0.0 Windows Vista -- 3.0.1 Build: 4.1.2.122 Setup: S0001 | Foster Tsai | First Release |

| 2007/01/29 | Utility: V1.00 (Build122-S0002) Driver: Windows 98SE/ME/2000/XP/XP x64 -- 1.2.0.0 Windows Vista -- 3.0.1 Build Version: 4.1.2.122 Setup Version: S0002 | Foster Tsai | - Modify Utility Security EAP Options in Vista - Fix Utility can’t modified profile under Vista - Fixed Utility auto start under Vista |
| --- | --- | --- | --- |
| 2007/02/08 | Utility: V1.00 (Build122-S0003) Driver: Windows 98SE/ME/2000/XP/XP x64 -- 1.2.0.0 Windows Vista -- 3.0.1 Build Version: 4.1.2.122 Setup Version: S0003 | Foster Tsai | - Remove build by DDK string. - Modified idle display rule. - Fixed Ad-Hoc channel range, Ad-Hoc profile in vista. - Fixed standby mode under vista - Service update for PSK slow connecting. |
| 2007/03/05 | Utility: V1.00 (Build305-S0004) Driver: Windows 2000/XP/98SE/ME – 1.2.0.0 Windows XP(64bit) – 1.2.0.0 Windows Vista – 3.0.1 | Foster Tsai | - Modified SSID can’t input special character (“&, <”) etc under Vista. - Install Shield change path fail. - Add scroll bar for profile list to long. |

| Date | Release Version | RD | Solved Bug Description |
| --- | --- | --- | --- |
| 2007/03/30 | Utility: V5.10 (Build61426-S0017)(15 Lang) Driver: Windows 2000/XP/98SE/ME – 1.01.01.0000 Windows Vista – 2.0.3.0 | Peja Hus | - Support WMM. - Support ISAREL channel 1~13. - Support WMM Power Save. - Fix wireless mode setting issue. - Limit username and password to 64 characters. - Driver V1.1.1.0 pass WHQL. - Support Vista. - For first time rebooting, D-Link Wizard will be automatically launching while no profile exists. - Support 18 languages in Installation Process & Utility: Chinese(Simplified), Chinese(Traditional), Czech, Danish, Dutch, English, Finnish, French, Herman, Greek, Hungarian, Italian, Norwegian, Polish, Portuguese, Russian, Spanish, Swedish. - Support Only 15 languages in D-Link Wizard: Czech, Danish, Dutch, English,Finnish, French, Herman, Greek, Hungarian, Italian, Norwegian, Polish, Portuguese, Spanish, Swedish. |

| Date | Release Version | RD | Solved Bug Description |
| --- | --- | --- | --- |
| 2007/04/03 | Utility: V5.10 (Build61614-S0018) Driver: Windows 2000/XP/98SE/ME – 1.01.01.0000 Windows Vista – 2.0.3.0 | Peja Hus Kevin Kao | 1. Support Only 15 languages in D-Link Wizard: Czech, Danish, Dutch, English, Finnish, French, Herman, Greek, Hungarian, Italian, Norwegian, Polish, Portuguese, Spanish, Swedish. 2. Divided 15 Languages version and 18 Languages version : 15L: Language List 18L: No language list |
| 2007/04/04 | Utility: V5.10 (Build61614-S0019) Driver: Windows 2000/XP/98SE/ME – 1.01.01.0000 Windows Vista – 2.0.3.0 | Peja Hus Kevin Kao | Divided 15 Languages version and 18 Languages version : 15L: Language List 18L: No language list |
| 2007/04/14 | Utility: V5.10 (Build61614-S0020) (15 Lang) Driver: Windows 2000/XP/98SE/ME – 1.01.01.0000 Windows Vista – 2.0.3.0 | Peja Hus Kevin Kao | - Fix Wizard wep profile cause Utility hang issue. - Fix keyboard selection not be stored in Wizard site survey list. - Fix missing character in taskbar tooltip text. |
| 2007/04/19 | Utility: V5.10 (Build61614-S00021) (15 Lang) Driver: Windows 2000/XP/ XP(64bit)/98SE/ME – 1.1.1.0 Windows Vista – 2.0.3.0 | Peja Hus Kevin Kao | - Fix Wizard supported limited Encryption and Cipher Type. - Zero Configuration would be restored after uninstall. |
| 2007/04/24 | Utility: V5.10 (BuildS0021) (18 Lang) Driver: Windows 2000/XP/ XP(64bit)/98SE/ME – 1.1.1.0 Windows Vista – 2.0.3.0 | Peja Hus Kevin Kao | Add three language: - Traditional Chinese - Simplified Chinese - Russian |
| 2007/04/24 | Utility: V5.10 (BuildS0022) (18 Lang) Driver: Windows 2000/XP/ XP(64bit)/98SE/ME – 1.1.1.0 Windows Vista – 2.0.3.0 | Peja Hus | Fix Russian installer string error. |
| 2007/04/26 | Utility: V5.10 (BuildS0023) (18 Lang) Driver: Windows 2000/XP/ XP(64bit)/98SE/ME – 1.1.1.0 Windows Vista – 2.0.3.0 | Peja Hus | Update Traditional Chinese/Simplified Chinese/Russian strings in installer and wizard. |
| 2007/04/27 | Utility: V5.10 (BuildS0024) (18 Lang) Driver: Windows 2000/XP/ XP(64bit)/98SE/ME – 1.1.1.0 Windows Vista – 2.0.3.0 | Peja Hus | - Fix inconsistent TC /SC strings. - Add TC/SC/Russian Wizard strings. (Currently support 18 L) |

Product Test History

| Release Date | Lab-Out Driver Version | Tester | Note |
| --- | --- | --- | --- |
| 2006/01/16 | Utility Version: V5.10 Driver Version: 1.0.3.0 Ralink Build Version: 3.3.1.51123 Setup Version: S0006 | Daby | - Old H/W and New Driver - Support OS: Windows98SE, Win ME, Win2K(SP4), Win XP(SP2), WinXP64bit 3. Support WMM function |
| 2006/05/15 | Utility Version: v 5.10 Driver Version: 1.01.00.0000 Build Version:3.3.1.60511 Setup Version: S0008 | Daby | 1.Ralink update the new driver 1.1.0.0. 2.Change the H/W to RoHS. |
| 2006/08/01 | Utility Version: v 5.10 (4OS) v1.00 (X64) Driver Version: 1.01.01.0000 Build Version:3.3.1.60728 Setup Version: S0012 | Daby | - Support WMM. - Support ISAREL channel 1~13. - Support WMM Power Save. - Limit username and password to 64 characters. |
| 2006/08/09 | UtilityVersion: v 5.10 (4OS) v1.00 (X64) DriverVersion: 1.01.01.0000 Build Version:3.3.1.60728 Setup Version: S0013 | Daby | Fix bug: - Copy Right change to 2006 - Install driver error issue - Setup version release to S0013 |
| 2007/04/03 | Utility: V5.10 (Build61426-S0017) Driver: Windows: 2000/XP/98SE/ME – 1.01.01.0000 Windows Vista – 2.0.3.0 | Daby | - Vista driver and throughput function test |
| 2007/04/19 | Utility: V5.10 (Build61614-S0021) Driver: Windows: 2000/XP/98SE/ME – 1.01.01.0000 Windows Vista – 2.0.3.0 | jimmy | - Test 6OS function: - Install driver - ICMP packets sending - Uninstall driver - Check Multi-Language (15ML) - Recheck issues occurring in DWL-G510 on DWL-G630 (Bug ID#1, #2, #3) |
| 2007/04/24 | Utility: V5.10 (S0021)(18 Lang) Driver: Windows: 2000/XP/98SE/ME – 1.01.01.0000 Windows Vista – 2.0.3.0 | jimmy | Check three language - Traditional Chinese - Simplified Chinese - Russian |
| 2007/04/25 | Utility: V5.10 (S0022)(18 Lang) Driver: Windows: 2000/XP/98SE/ME – 1.01.01.0000 Windows Vista – 2.0.3.0 | jimmy | ReCheck Russian installer string error |
| 2007/04/27 | Utility: V5.10 (S0023)(18 Lang) Driver: Windows: 2000/XP/98SE/ME – 1.01.01.0000 Windows Vista – 2.0.3.0 | jimmy | Recheck TC/SC/Russian strings in installer and wizard. |
| 2007/04/27 | Utility: V5.10 (S0024)(18 Lang) Driver: Windows: 2000/XP/98SE/ME – 1.01.01.0000 Windows Vista – 2.0.3.0 | jimmy | Recheck TC/SC/Russian strings in installer and wizard. |

Test Result

The test request note is shown as the following file:

1. Bug ID #1: WMM function is unavailable in Windows Vista x86 (Function Error)

The WMM function is unavailable in Windows Vista x86.  According to the driver options in Windows Vista Network Manager, the WMM function is enabled in default.  But when we captured the packets launched from DUT, the WME field in packets did not appear and therefore this function is unavailable.

=>RD :Driver does not support WMM function in Vista OS.

Table of Contents

1. Introduction Of Test Product(Present by Product Manager)

2. Basic Function Test	12

2-1 Wireless Band	12

2-2 SSID	12

2-3 SSID Broadcast	13

2-4 Channel	13

2-5 Protocol	13

2-6 Dynamic Rate shifting	13

2-7 Fragment Length	14

2-8 RTS Length	14

2-9 Roaming	14

2-10 DHCP	14

2-11 Login in Windows domain controller	14

2-12 OS Standby Test	15

2-13 Win XP/XP64bit/Vista for Zero Configuration Utility-Option	15

2-14 Driver Modifying Protection with a Normal User	15

2-15 D-Link Utility Option	15

2-16 PC/NB Compatibility	17

3.Security Test	18

3-1 WEP	18

3-2 WPA-PSK	20

3-3 WPA2-PSK	20

3-4 WPA	20

3-5 WPA2-EAP	21

3-6 802.1X	21

4. Throughput and Compatibility Test	22

4-1 Card Compatibility Test (Ad-Hoc Mode)	22

4-1-1 Ad-Hoc (Windows XP SP2)	22

4-1-2 Ad-Hoc (Windows XP64)	22

4-1-3 Ad-Hoc (Windows Vista x86)	23

4-2 AP Compatibility Test (Infrastructure Mode)	23

4-2-1 802.11G (Windows XP SP2)	23

4-2-2 802.11g (Windows XP64)	23

4-2-3 802.11g (Windows Vista x86)	24

4-2-4 802.11b (Windows Vista x86)	24

4-3 Channel (Infrastructure Mode)	24

4-3-1 802.11G (Windows XP SP2)	25

4-3-2 802.11g (Windows XP64)	25

4-3-3 802.11g (Windows Vista x86)	25

4-3-4 802.11b (Windows Vista x86)	26

4-3-5 Ad-Hoc (Windows Vista x86)	26

5. Endurance Test	27

5-1 Basic Function Endurance	27

5-1 Basic Endurance	27

5-1-1 Infrastructure mode	27

5-1-2 Ad-Hoc mode	27

5-1-3 Infrastructure mode with and without PSP mode	27

5-2 Security Endurance	28

5-2-1 Endurance Test (Infrastructure mode with and without Encryption)	28

5-2-2 Infrastructure mode	28

5-2-3 Ad-hoc mode	28

6. Other Related Test	29

6-1 WIFI Pretest	29

6-2 Firewall	29

6-3 Multi-Language User Interface Test	29

6-4 WMM Function Test	30

6-4-1 Windows XP SP2	30

6-4-2 Windows XP X64	32

6-5 WMM Power Save Function Test	34

6-6 Microsoft Certify Test	35

# 2. Basic Function Test

Test Environment

Station: ASUS P5GD2-X [OS: Windows Vista x86]

ASUS P4B [OS: 5OS]

AP: DWL-8200 [F/W: v121.rc067.t624us]

File Size: 600MB

## 2-1 Wireless Band

| Description | Result (Pass/ Fail) |
| --- | --- |
| 802.11b (It can support 802.11b only) | Pass |
| 802.11g (It can support 802.11g only) | Pass |
| 802.11b/g mixed | Pass |

## 2-2 SSID

| Description | Result (Pass/ Fail) |
| --- | --- |
| It supports 32 characters | Pass |
| STA SSID:12345678901234567890123456789011 AP SSID:12345678901234567890123456789012 | Pass |
| STA SSID:~!@#$%^&*() AP SSID:~!@#$%^&*() | Pass |
| STA SSID:NULL AP SSID:~!@#$%^&*() | Pass |
| STA SSID:123 45 AP SSID:12345 | Pass |
| “Blank” character | Pass |

## 2-3 SSID Broadcast

| Description | Result (Pass/ Fail) |
| --- | --- |
| Enable | Pass |
| Disable | Pass |

## 2-4 Channel

| Channel | 802.11b | 802.11g | Ad-Hoc |
| --- | --- | --- | --- |
|  | Result (Pass/Fail) | Result (Pass/Fail) | Result (Pass/Fail) |
| 1 | Pass | Pass | Pass |
| 2 | Pass | Pass | Pass |
| 3 | Pass | Pass | Pass |
| 4 | Pass | Pass | Pass |
| 5 | Pass | Pass | Pass |
| 6 | Pass | Pass | Pass |
| 7 | Pass | Pass | Pass |
| 8 | Pass | Pass | Pass |
| 9 | Pass | Pass | Pass |
| 10 | Pass | Pass | Pass |
| 11 | Pass | Pass | Pass |
| 12 | Pass | Pass | Pass |
| 13 | Pass | Pass | Pass |

## 2-5 Protocol

| OS Protocol | Win98 SE | Win ME | Win 2000 SP4 | Win XP SP2 | Win XPx64 | VistaX86 |
| --- | --- | --- | --- | --- | --- | --- |
| TCP/IP | Pass | Pass | Pass | Pass | Pass | Pass |
| NetBEUI | Pass | Pass | Pass | Pass | N/A | N/A |
| IPX+ | Pass | Pass | Pass | Pass | Pass | N/A |

- Windows Vista does not support IPX and NETBEUI protocol.

## 2-6 Dynamic Rate shifting

| Description | Result (Pass/ Fail) |
| --- | --- |
| Auto | Pass |

## 2-7 Fragment Length

| Description | Result (Pass/ Fail) |
| --- | --- |
| Default-2346 | Pass |
| 512 | Pass |
| 256 | Pass |

## 2-8 RTS Length

| Description | Result (Pass/ Fail) |
| --- | --- |
| Default-2346 | Pass |
| 512 | Pass |
| 256 | Pass |

## 2-9 Roaming

| Description | Result (Pass/ Fail) |
| --- | --- |
| Test a station’s ability to roam between two access points | Pass |

## 2-10 DHCP

| Description | Result (Pass/ Fail) |
| --- | --- |
| Card can get IP from the DHCP server through an AP | Pass |

## 2-11 Login in Windows domain controller

Server: Windows 2000 advance server as a Domain Controller

| OS | Security | Result (Pass/Fail) |
| --- | --- | --- |
| Win 98 SE | 802.1X + EAP-TTLS with MSCHAPV2 | Pass |
| Win 2000 SP4 | Open System with Non-Encryption | Pass |
| Win XP SP2 | Open System + WEP 128 bits Hex Key | Pass |
| Win XP x64 | 802.1X + EAP-TTLS with MSCHAPV2 | Pass |
| Win Vistax86 | N/A | N/A |

- Windows 2000 Server Login is not supported by Windows Vista.

## 2-12 OS Standby Test

| OS | Result (Pass/Fail) |
| --- | --- |
| WinXP64 SP1 | Pass |
| Win XP Pro. SP2 | Pass |
| Win2000 Pro. SP4 | Pass |
| Win ME | Pass |
| Win98 SE | Pass |
| Win Vistax86 | Pass |

## 2-13 Win XP/XP64bit/Vista for Zero Configuration Utility-Option

| Description | Result (Pass/Fail) |
| --- | --- |
| Infrastructure | Pass |
| Ad-Hot | Pass |

## 2-14 Driver Modifying Protection with a Normal User

| Description | Result (Pass/Fail) |
| --- | --- |
| Win XP x64 Edition | Pass |
| Win XP SP2 | Pass |
| Win 2000 SP4 | Pass |

## 2-15 D-Link Utility Option

| Item | Result (Pass/Fail) |  |
| --- | --- | --- |
|  | Other OS | Windows Vista |
| Wizard: |  |  |
| Enter Wireless Network Name | Pass | Pass |
| Scan | Pass | Pass |
| Wireless Networks: |  |  |
| Refresh | Pass | Pass |
| Connect | Pass | Pass |
| Activate | Pass | Pass |
| My Wireless Networks |  |  |
| New | Pass | Pass |
| Modify | Pass | Pass |
| Delete | Pass | Pass |
| Activate | Pass | Pass |

| Profile Settings: Basic Settings: |  |  |
| --- | --- | --- |
| Profile Name | Pass | Pass |
| SSID | Pass | Pass |
| Networks Type (Infrastructure) | Pass | Pass |
| Networks Type (Ad-Hoc) | Pass | Pass |
| Set Security Option: WEP: |  |  |
| Open | Pass | Pass |
| Shared | Pass | Pass |
| Auto | Pass | Pass |
| WPA/WPA2 PassPhase: |  |  |
| TKIP | Pass | Pass |
| AES | Pass | Pass |
| Auto | Pass | Pass |
| WPA/WPA2: |  |  |
| TKIP | Pass | N/A |
| AES | Pass | N/A |
| Auto | Pass | N/A |
| Advanced Config | Pass | Pass |
| 802.1x: |  |  |
| Advanced Config | Pass | Pass |
| None: |  |  |
| None Security | Pass | Pass |
| Support  About: |  |  |
| Driver Version | Pass | Pass |
| Utility Version | Pass | Pass |
| Mac Address | Pass | Pass |
| Build Info: |  |  |
| Build Version | Pass | Pass |
| Reg Domain | Pass | Pass |
| Firmware | Pass | Pass |
| Setup Version | Pass | Pass |
| Profile Details: |  |  |
| Network Name | Pass | Pass |
| Network Type | Pass | Pass |
| Security | Pass | Pass |

- Now the driver for Windows Vista x86 uses Windows’ supplicant to create WPA-EAP, WPA2-EAP and 802.1x connections.  Because Windows Vista does not provide APIs to support vendors to call their supplicant and fill in connection parameters, therefore the setting parameters in UI are unavailable in Windows Vista x86 and users need to re-input their connection parameters with the pop-up dialog box from Windows Vista.

## 2-16 PC/NB Compatibility

Test Environment

OS: Win2k SP4, Win_ XP SP2

Test Program: FTP for windows

| Ser No. | Model Name | CPU | BIOS | MB Chipset | BUS | USB Chip | OS | Test Conclusion (Pass/Fail) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TLD2097 | ASUS P5P800 | Pentium 4 3.0G | AMI BIOS | Intel865G | PCI | IntelI 82801 | Windows XP SP2 | Put 600MB | Pass |
|  |  |  |  |  |  |  |  | Get 600MB | Pass |
|  |  |  |  |  |  |  |  | S1 Standby | Pass |
|  |  |  |  |  |  |  |  | S4 Hibernate | Pass |
|  |  |  |  |  |  |  |  | Enable/Disable | Pass |
|  |  |  |  |  |  |  |  | Hot swap | Pass |
|  |  |  |  |  |  |  |  | Uninstall | Pass |
| TLD2N33 | Toshiba Tecra A3/S2 | Intel Pentium M 1.6 GHz | Intel 945PM | Intel 82801 Family | CardBus | Intel 82801(ICH7 Family) | Windows XP SP2 | Put 600MB | Pass |
|  |  |  |  |  |  |  |  | Get 600MB | Pass |
|  |  |  |  |  |  |  |  | S1 Standby | Pass |
|  |  |  |  |  |  |  |  | S4 Hibernate | Pass |
|  |  |  |  |  |  |  |  | Enable/Disable | Pass |
|  |  |  |  |  |  |  |  | Hot swap | Pass |
|  |  |  |  |  |  |  |  | Uninstall | Pass |
| TLD2N36 | Acer Aspire 5580 | Intel Centrino Core2 Duo 1.66GHz | Intel 945GM | Intel 82801G (ICH7 Family) | Cardbus | TI PCIxx12 | Windows XP SP2 | Put 600MB | Pass |
|  |  |  |  |  |  |  |  | Get 600MB | Pass |
|  |  |  |  |  |  |  |  | S1 Standby | Pass |
|  |  |  |  |  |  |  |  | S4 Hibernate | Pass |
|  |  |  |  |  |  |  |  | Enable/Disable | Pass |
|  |  |  |  |  |  |  |  | Hot swap | Pass |
|  |  |  |  |  |  |  |  | Uninstall | Pass |
|  | ASUS (PUNDIT) AB-2800 | Pentium4R8L 2.8Ghz | AMI | ITE IT8712F-A | PCI | USB2.0 | Windows XP SP2 | Put 600MB | Pass |
|  |  |  |  |  |  |  |  | Get 600MB | Pass |
|  |  |  |  |  |  |  |  | S1 Standby | Pass |
|  |  |  |  |  |  |  |  | S4 Hibernate | Pass |
|  |  |  |  |  |  |  |  | Enable/Disable | Pass |
|  |  |  |  |  |  |  |  | Hot swap | Pass |
|  |  |  |  |  |  |  |  | Uninstall | Pass |

# 3.Security Test

Test Environment

Station: ASUS P5GD2-X [OS: Windows XP SP2, Windows Vista x86]

AP: DWL-8200 [F/W: v121.rc067.t624us], Cisco 1200 [F/W: 12.3(2)JA]

Tools: Ping Command, AiroPeek

Channel: 9

## 3-1 WEP

| Item | Authentication | Key type | Key Size | Valid Key | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| 1 | Open System | Hex | 64 Bits | First | Pass |
| 2 | Open System | Hex | 64 Bits | Second | Pass |
| 3 | Open System | Hex | 64 Bits | Third | Pass |
| 4 | Open System | Hex | 64 Bits | Fourth | Pass |
| 5 | Open System | Hex | 128 Bits | First | Pass |
| 6 | Open System | Hex | 128 Bits | Second | Pass |
| 7 | Open System | Hex | 128 Bits | Third | Pass |
| 8 | Open System | Hex | 128 Bits | Fourth | Pass |
| 9 | Open System | Hex | 152 Bits | First | Don’t support |
| 10 | Open System | Hex | 152 Bits | Second | Don’t support |
| 11 | Open System | Hex | 152 Bits | Third | Don’t support |
| 12 | Open System | Hex | 152 Bits | Fourth | Don’t support |
| 13 | Open System | ASCII | 64 Bits | First | Pass |
| 14 | Open System | ASCII | 64 Bits | Second | Pass |
| 15 | Open System | ASCII | 64 Bits | Third | Pass |
| 16 | Open System | ASCII | 64 Bits | Fourth | Pass |
| 17 | Open System | ASCII | 128 Bits | First | Pass |
| 18 | Open System | ASCII | 128 Bits | Second | Pass |
| 19 | Open System | ASCII | 128 Bits | Third | Pass |
| 20 | Open System | ASCII | 128 Bits | Fourth | Pass |
| 21 | Open System | ASCII | 152 Bits | First | Don’t support |
| 22 | Open System | ASCII | 152 Bits | Second | Don’t support |
| 23 | Open System | ASCII | 152 Bits | Third | Don’t support |
| 24 | Open System | ASCII | 152 Bits | Fourth | Don’t support |
| 25 | Shared Key | Hex | 64 Bits | First | Pass |
| 26 | Shared Key | Hex | 64 Bits | Second | Pass |
| 27 | Shared Key | Hex | 64 Bits | Third | Pass |
| 28 | Shared Key | Hex | 64 Bits | Fourth | Pass |
| 29 | Shared Key | Hex | 128 Bits | First | Pass |

| Item | Authentication | Key type | Key Size | Valid Key | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| 30 | Shared Key | Hex | 128 Bits | Second | Pass |
| 31 | Shared Key | Hex | 128 Bits | Third | Pass |
| 32 | Shared Key | Hex | 128 Bits | Fourth | Pass |
| 33 | Shared Key | Hex | 152 Bits | First | Don’t support |
| 34 | Shared Key | Hex | 152 Bits | Second | Don’t support |
| 35 | Shared Key | Hex | 152 Bits | Third | Don’t support |
| 36 | Shared Key | Hex | 152 Bits | Fourth | Don’t support |
| 37 | Shared Key | ASCII | 64 Bits | First | Pass |
| 38 | Shared Key | ASCII | 64 Bits | Second | Pass |
| 39 | Shared Key | ASCII | 64 Bits | Third | Pass |
| 40 | Shared Key | ASCII | 64 Bits | Fourth | Pass |
| 41 | Shared Key | ASCII | 128 Bits | First | Pass |
| 42 | Shared Key | ASCII | 128 Bits | Second | Pass |
| 43 | Shared Key | ASCII | 128 Bits | Third | Pass |
| 44 | Shared Key | ASCII | 128 Bits | Fourth | Pass |
| 45 | Shared Key | ASCII | 152 Bits | First | Don’t support |
| 46 | Shared Key | ASCII | 152 Bits | Second | Don’t support |
| 47 | Shared Key | ASCII | 152 Bits | Third | Don’t support |
| 48 | Shared Key | ASCII | 152 Bits | Fourth | Don’t support |
| 49 | Open/Shared | Hex | 64 Bits | First | Pass |
| 50 | Open/Shared | Hex | 64 Bits | Second | Pass |
| 51 | Open/Shared | Hex | 64 Bits | Third | Pass |
| 52 | Open/Shared | Hex | 64 Bits | Fourth | Pass |
| 53 | Open/Shared | Hex | 128 Bits | First | Pass |
| 54 | Open/Shared | Hex | 128 Bits | Second | Pass |
| 55 | Open/Shared | Hex | 128 Bits | Third | Pass |
| 56 | Open/Shared | Hex | 128 Bits | Fourth | Pass |
| 57 | Open/Shared | Hex | 152 Bits | First | Don’t support |
| 58 | Open/Shared | Hex | 152 Bits | Second | Don’t support |
| 59 | Open/Shared | Hex | 152 Bits | Third | Don’t support |
| 60 | Open/Shared | Hex | 152 Bits | Fourth | Don’t support |
| 61 | Open/Shared | ASCII | 64 Bits | First | Pass |
| 62 | Open/Shared | ASCII | 64 Bits | Second | Pass |
| 63 | Open/Shared | ASCII | 64 Bits | Third | Pass |
| 64 | Open/Shared | ASCII | 64 Bits | Fourth | Pass |
| 65 | Open/Shared | ASCII | 128 Bits | First | Pass |
| 66 | Open/Shared | ASCII | 128 Bits | Second | Pass |
| 67 | Open/Shared | ASCII | 128 Bits | Third | Pass |
| 68 | Open/Shared | ASCII | 128 Bits | Fourth | Pass |
| 69 | Open/Shared | ASCII | 152 Bits | First | Don’t support |
| 70 | Open/Shared | ASCII | 152 Bits | Second | Don’t support |
| 71 | Open/Shared | ASCII | 152 Bits | Third | Don’t support |
| 72 | Open/Shared | ASCII | 152 Bits | Fourth | Don’t support |

## 3-2 WPA-PSK

| Item | Cipher Type | Result (Pass/Fail) |
| --- | --- | --- |
| 1 | TKIP | Pass |
| 2 | AES | Pass |
| 3 | It supports 8~64 bits | Pass |

## 3-3 WPA2-PSK

| Item | Cipher Type | Result (Pass/Fail) |
| --- | --- | --- |
| 1 | TKIP | Pass |
| 2 | AES | Pass |
| 3 | It supports 8~64 bits | Pass |

## 3-4 WPA

| Item | Cipher | EAP | Radius | Result (Pass/Fail) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Vista x86 | Other OS |
| 1 | TKIP | EAP-TLS | IAS | Pass | Pass |
| 2 | TKIP | PEAP(EAP-MSCHAPV2) | IAS | Pass | Pass |
| 3 | TKIP | EAP-TLS | Odyssey | N/A | Pass |
| 5 | TKIP | EAP-TTLS(PAP) | Odyssey | N/A | Pass |
| 6 | TKIP | EAP-TTLS(MSCHAP) | Odyssey | N/A | Pass |
| 7 | TKIP | EAP-TTLS(MSCHAPV2) | Odyssey | N/A | Pass |
| 8 | TKIP | PEAP(EAP-MSCHAPV2) | Odyssey | Pass | Pass |
| 9 | AES | EAP-TLS | IAS | Pass | Pass |
| 10 | AES | PEAP(EAP-MSCHAPV2) | IAS | Pass | Pass |
| 11 | AES | EAP-TLS | Odyssey | N/A | Pass |
| 12 | AES | EAP-TTLS(PAP) | Odyssey | N/A | Pass |
| 13 | AES | EAP-TTLS(MSCHAP) | Odyssey | N/A | Pass |
| 14 | AES | EAP-TTLS(MSCHAPV2) | Odyssey | N/A | Pass |
| 15 | AES | PEAP(EAP-MSCHAPV2) | Odyssey | Pass | Pass |
| 16 | Auto | EAP-TLS | IAS | Pass | Pass |
| 17 | Auto | PEAP(EAP-MSCHAPV2) | IAS | Pass | Pass |
| 18 | Auto | EAP-TLS | Odyssey | N/A | Pass |
| 19 | Auto | EAP-TTLS(PAP) | Odyssey | N/A | Pass |
| 20 | Auto | EAP-TTLS(MSCHAP) | Odyssey | N/A | Pass |
| 21 | Auto | EAP-TTLS(MSCHAPV2) | Odyssey | N/A | Pass |
| 22 | Auto | PEAP(EAP-MSCHAPV2) | Odyssey | Pass | Pass |

- The EAP-TLS authentication in Windows Vista cannot work with Odyssey Server 2.0.

- The LEAP, EAP-TTLS authentications are not supported by Windows Vista API.

## 3-5 WPA2-EAP

| Item | Cipher | EAP | Radius | Result (Pass/Fail) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Vista x86 | Other OS |
| 1 | TKIP | EAP-TLS | IAS | Pass | Pass |
| 2 | TKIP | PEAP(EAP-MSCHAPV2) | IAS | Pass | Pass |
| 3 | TKIP | EAP-TLS | Odyssey | N/A | Pass |
| 4 | TKIP | LEAP | Odyssey | N/A | Pass |
| 5 | TKIP | EAP-TTLS(PAP) | Odyssey | N/A | Pass |
| 6 | TKIP | EAP-TTLS(MSCHAP) | Odyssey | N/A | Pass |
| 7 | TKIP | EAP-TTLS(MSCHAPV2) | Odyssey | N/A | Pass |
| 8 | TKIP | PEAP(EAP-MSCHAPV2) | Odyssey | Pass | Pass |
| 9 | AES | EAP-TLS | IAS | Pass | Pass |
| 10 | AES | PEAP(EAP-MSCHAPV2) | IAS | Pass | Pass |
| 11 | AES | EAP-TLS | Odyssey | N/A | Pass |
| 12 | AES | EAP-TTLS(PAP) | Odyssey | N/A | Pass |
| 13 | AES | EAP-TTLS(MSCHAP) | Odyssey | N/A | Pass |
| 14 | AES | EAP-TTLS(MSCHAPV2) | Odyssey | N/A | Pass |
| 15 | AES | PEAP(EAP-MSCHAPV2) | Odyssey | Pass | Pass |
| 16 | Auto | EAP-TLS | IAS | Pass | Pass |
| 17 | Auto | PEAP(EAP-MSCHAPV2) | IAS | Pass | Pass |
| 18 | Auto | EAP-TLS | Odyssey | N/A | Pass |
| 19 | Auto | EAP-TTLS(PAP) | Odyssey | N/A | Pass |
| 20 | Auto | EAP-TTLS(MSCHAP) | Odyssey | N/A | Pass |
| 21 | Auto | EAP-TTLS(MSCHAPV2) | Odyssey | N/A | Pass |
| 22 | Auto | PEAP(EAP-MSCHAPV2) | Odyssey | Pass | Pass |

- The EAP-TLS authentication in Windows Vista cannot work with Odyssey Server 2.0.

- The EAP-TTLS authentications are not supported by Windows Vista API.

## 3-6 802.1X

| Item | Authentication Mode | Radius Server | Cipher Type | EAP-Type | Result (Pass/Fail) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Vista x86 | Other OS |
| 1 | Open | Odyssey server 2.0 | WEP | EAP-TLS | N/A | Pass |
| 2 | Open | Odyssey server 2.0 | WEP | LEAP | N/A | Pass |
| 3 | Open | Odyssey server 2.0 | WEP | EAP-TTLS(PAP) | N/A | Pass |
| 4 | Open | Odyssey server 2.0 | WEP | EAP-TTLS(MSCHAP) | N/A | Pass |
| 5 | Open | Odyssey server 2.0 | WEP | EAP-TTLS(MSCHAPV2) | N/A | Pass |
| 6 | Open | Odyssey server 2.0 | WEP | PEAP (EAP-MSCHAP V2) | Pass | Pass |
| 7 | Open | IAS | WEP | PEAP (EAP-MSCHAPv2) | Pass | Pass |
| 8 | Open | IAS | WEP | EAP-TLS | Pass | Pass |

- The EAP-TLS authentication in Windows Vista cannot work with Odyssey Server 2.0.

- The LEAP, EAP-TTLS authentications are not supported by Windows Vista API.

# 4. Throughput and Compatibility Test

Test Environment

Server: ASUS (PUNDIT) AB-2800 R4R8L-2.8G [OS: Windows XP SP2]

Station: ASUS P5P800-VM [OS: Windows XP SP2, Windows XP64]

Station: ASUS P5B-VM [OS: Windows Vista x86]

Test tool: Chariot console 5.9 (3186), Chariot endpoint 6.40

Default Channel: 6

Place: Wireless Shield Room II

## 4-1 Card Compatibility Test (Ad-Hoc Mode)

### 4-1-1 Ad-Hoc (Windows XP SP2)

File Size: 1MB

| Vendors Card | Version | Chariot Unit: Mbps |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | No WEP |  | WEP-128bits |  |
|  |  | TX | RX | TX | RX |
| ##### Buffalo CB54G | 1.4.20.5 | 5.498 | 5.913 | 5.439 | 5.874 |
| Cisco CB21AG | 3.0.0.104 | 5.279 | 5.392 | 5.454 | 5.397 |
| Orinoco | 3.1.2.19 | 5.161 | 5.063 | 5.165 | 5.195 |
| DWL-AG660 | 4.1.2.147 | 5.275 | 5.347 | 5.239 | 5.363 |

### 4-1-2 Ad-Hoc (Windows XP64)

File Size: 1MB

| Vendors Card | Version | Chariot Unit: Mbps |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | No WEP |  | WEP-128bits |  |
|  |  | TX | RX | TX | RX |
| ##### Buffalo CB54G | 1.4.20.5 | 5.702 | 6.021 | 5.440 | 5.876 |
| Cisco CB21AG | 3.0.0.104 | 5.275 | 5.405 | 5.262 | 5.371 |
| Orinoco | 3.1.2.19 | 5.201 | 5.112 | 5.148 | 5.060 |
| DWL-AG660 | 4.1.2.147 | 5.287 | 5.355 | 5.256 | 5.356 |

### 4-1-3 Ad-Hoc (Windows Vista x86)

File Size: 1MB

| Vendors Card | Version | Chariot Unit: Mbps |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | No WEP |  | WEP-128bits |  |
|  |  | TX | RX | TX | RX |
| ##### Buffalo CB54G | 1.4.20.5 | 5.601 | 5.945 | 5.577 | 5.909 |
| Cisco CB21AG | 3.0.0.104 | 5.247 | 5.244 | 5.330 | 5.325 |
| Orinoco | 3.1.2.19 | 5.569 | 5.313 | 5.302 | 5.542 |
| DWL-AG660 | 4.1.2.147 | 5.245 | 5.298 | 5.451 | 5.269 |

## 4-2 AP Compatibility Test (Infrastructure Mode)

### 4-2-1 802.11G (Windows XP SP2)

File Size: 1MB

| AP | Version | Chariot Unit: Mbps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | No WEP |  | WEP-128bits |  | WPA-PSK (TKIP) |  |
|  |  | TX | RX | TX | RX | TX | RX |
| D-Link DWL-8200 | 1.21 | 22.027 | 20.550 | 21.798 | 21.758 | 22.113 | 22.358 |
| DWL-7100 | 2.10 | 20.690 | 22.436 | 22.076 | 22.682 | 21.736 | 22.351 |
| Cameo DI-624 C3 | 2.70 | 21.917 | 22.265 | 21.724 | 22.388 | 21.722 | 22.140 |
| Cameo DI-784 | 2.42 | 2.055 | 21.607 | 21.867 | 22.491 | 21.864 | 21.724 |

### 4-2-2 802.11g (Windows XP64)

File Size: 1MB

| AP | Version | Chariot Unit: Mbps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | No WEP |  | WEP-128bits |  | WPA-PSK (TKIP) |  |
|  |  | TX | RX | TX | RX | TX | RX |
| D-Link DWL-8200 | 1.21 | 22.427 | 22.603 | 21.983 | 22.839 | 21.854 | 22.353 |
| Cisco 1200 | 12.3(2)JA | 21.369 | 22.138 | 21.686 | 21.671 | 21.031 | 21.390 |
| Cameo DI-624 C3 | 2.70 | 21.489 | 21.381 | 21.442 | 22.366 | 21.581 | 21.840 |
| Cameo DI-784 | 2.43 | 22.043 | 21.016 | 21.639 | 21.012 | 21.188 | 22.230 |

### 4-2-3 802.11g (Windows Vista x86)

File Size: 1MB

| AP | Version | Chariot Unit: Mbps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | No WEP |  | WPA-PSK (TKIP) |  | WPA2-PSK (AES) |  |
|  |  | TX | RX | TX | RX | TX | RX |
| D-Link DWL-8200 | 1.21 | 22.123 | 22.276 | 21.567 | 22.159 | 21.490 | 22.023 |
| Cameo DI-624 D2 | 4.03 | 22.140 | 22.271 | 21.306 | 21.762 | 21.515 | 21.932 |
| Cameo DI-784 | 2.44 beta | 21.976 | 22.326 | 21.692 | 21.879 | 21.581 | 21.874 |
| Cisco 1200 | 12.3(2)JA | 22.171 | 17.011 | 20.152 | 17.009 | 19.900 | 16.550 |

### 4-2-4 802.11b (Windows Vista x86)

File Size: 1MB

| AP | Version | Chariot Unit: Mbps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | No WEP |  | WPA-PSK (TKIP) |  | WPA2-PSK (AES) |  |
|  |  | TX | RX | TX | RX | TX | RX |
| D-Link DWL-8200 | 1.21 | 5.958 | 5.660 | 5.884 | 5.700 | 5.839 | 5.639 |
| Cisco 1200 | 12.3(2)JA | 6.434 | 6.693 | 6.276 | 6.366 | 6.170 | 5.957 |

## 4-3 Channel (Infrastructure Mode)

Test Environment

Server: ASUS (PUNDIT) AB-2800 R4R8L-2.8G [OS: Windows XP SP2]

Station: ASUS P5P800-VM [OS: Windows XP64]

Station: ASUS P4B800VM 3.0G [OS: Windows XP SP2]

Station: ASUS P5B-VM [OS: Windows Vista x86]

Test tool: Chariot console 5.9 (3186), Chariot endpoint 6.40

Place: Wireless Shield Room II

### 4-3-1 802.11G (Windows XP SP2)

File Size: 1MB

| AP | Version | Channel | Chariot Unit: Mbps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | No WEP |  | WEP-128bits |  | WPA-PSK (TKIP) |  |
|  |  |  | TX | RX | TX | RX | TX | RX |
| DI-624 C3 | 2.70 | 1 | 22.458 | 22.711 | 20.371 | 22.446 | 21.498 | 22.066 |
|  |  | 6 | 21.031 | 22.303 | 21.647 | 22.459 | 21.519 | 22.270 |
|  |  | 11 | 22.130 | 22.144 | 20.945 | 22.031 | 21.150 | 21.660 |

### 4-3-2 802.11g (Windows XP64)

File Size: 1MB

| AP | Version | Channel | Chariot Unit: Mbps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | No WEP |  | WEP-128bits |  | WPA-PSK (TKIP) |  |
|  |  |  | TX | RX | TX | RX | TX | RX |
| DI-624 C3 | 2.70 | 1 | 21.466 | 22.484 | 21.651 | 22.070 | 21.265 | 22.291 |
|  |  | 6 | 22.435 | 22.348 | 21.481 | 22.356 | 21.460 | 21.801 |
|  |  | 11 | 21.489 | 21.381 | 21.442 | 22.366 | 21.581 | 21.840 |

### 4-3-3 802.11g (Windows Vista x86)

File Size: 1MB

| AP | Version | Channel | Chariot Unit: Mbps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | No WEP |  | WPA-PSK (TKIP) |  | WPA2-PSK (AES) |  |
|  |  |  | TX | RX | TX | RX | TX | RX |
| DWL-8200 | 1.21 | 1 | 21.656 | 21.014 | 21.744 | 21.739 | 21.754 | 22.288 |
|  |  | 6 | 22.123 | 22.276 | 21.567 | 22.159 | 21.490 | 22.023 |
|  |  | 11 | 21.359 | 22.297 | 21.610 | 21.942 | 21.860 | 22.315 |
| Cisco 1200 | 12.3(2)JA | 1 | 21.420 | 17.227 | 19.965 | 17.018 | 21.848 | 16.932 |
|  |  | 6 | 22.171 | 17.011 | 20.152 | 17.009 | 19.900 | 16.550 |
|  |  | 11 | 21.079 | 15.054 | 19.837 | 14.831 | 20.819 | 15.589 |

### 4-3-4 802.11b (Windows Vista x86)

File Size: 1MB

| AP | Version | Channel | Chariot Unit: Mbps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | No WEP |  | WPA-PSK (TKIP) |  | WPA2-PSK (AES) |  |
|  |  |  | TX | RX | TX | RX | TX | RX |
| D-Link DWL-8200 | 1.21 | 1 | 5.304 | 5.272 | 5.303 | 5.298 | 5.254 | 5.217 |
|  |  | 6 | 5.316 | 5.293 | 5.262 | 5.256 | 5.273 | 5.234 |
|  |  | 11 | 5.289 | 5.293 | 5.291 | 5.210 | 5.256 | 5.218 |
| Cisco 1200 | 12.3(2)JA | 1 | 5.195 | 5.405 | 5.053 | 5.305 | 5.039 | 5.329 |
|  |  | 6 | 5.121 | 5.398 | 5.073 | 5.157 | 5.076 | 5.346 |
|  |  | 11 | 5.189 | 5.425 | 5.036 | 5.028 | 5.089 | 5.284 |

### 4-3-5 Ad-Hoc (Windows Vista x86)

File Size: 1MB

| Vendors Card | Version | Channel | Chariot Unit: Mbps |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | No WEP |  | WEP-128bits |  |
|  |  |  | TX | RX | TX | RX |
| ##### Buffalo CB54G | 1.4.20.5 | 1 | 5.585 | 5.944 | 5.576 | 5.907 |
|  |  | 6 | 5.601 | 5.945 | 5.577 | 5.909 |
|  |  | 11 | 5.569 | 5.903 | 5.576 | 5.918 |
| Cisco CB21AG | 3.0.0.104 | 1 | 5.333 | 5.414 | 5.370 | 5.427 |
|  |  | 6 | 5.247 | 5.244 | 5.330 | 5.325 |
|  |  | 11 | 5.531 | 5.545 | 5.549 | 5.429 |

# 5. Endurance Test

## 5-1 Basic Function Endurance

Test Environment

Test tool: Chariot 5.0, Serv-U 2.5, FTP Command

Test Method: FTP => Run a 600MB TX and 600MB RX simultaneously in open side over 12 hours.

Chariot => Run a 100MB TX and 100MB RX Chariot test pair in shield room over 12 hours.

Test Place: Lab (Open Side), Shield Room

## 5-1 Basic Endurance

### 5-1-1 Infrastructure mode

| Item | PC/Note Book | OS | AP | Channel | Method | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | MSI-K8T Neo2 | Windows XP64 | DWL-8200 | 11 | FTP | Pass |
| 2 | ASUS L3400C | Windows XP SP2 | DWL-8200 | 11 | FTP | Pass |
| 3 | ASUS P5B-VM | Windows Vista x86 | DWL-8200 | 6 | Chariot | Pass |

### 5-1-2 Ad-Hoc mode

| Item | PC/Note Book | OS | Test Card | Channel | Method | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | ASUS L4000L | Windows 98SE | Cisco CB21AG | 1 | Chariot | Pass |
|  | Acer Aspire 5583 | Windows Vista x86 | DWL-510 |  |  |  |
| 2 | ASUS P5B-VM | Windows Vista x86 | DWL-630 | 6 | Chariot | Pass |
|  | ASUS P5P800 | Windows Vista x86 | DWL-510 |  |  |  |

### 5-1-3 Infrastructure mode with and without PSP mode

| Item | PC/Note Book | OS | Channel | AP | PSP | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | ASUS P4T/W0A | Windows 2000 SP4 | 1 | DWL-8200 | Max save | Pass |

## 5-2 Security Endurance

Test Environment

File Size: 600MB

Utility: FTP command, Media Player
Run: Over 24 hrs

### 5-2-1 Endurance Test (Infrastructure mode with and without Encryption)

| Item | PC/Note Book | OS | AP | Encryption | Channel | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | ASUS P4T/WoA | Windows SP2 (WZC) | DWL-8200 | WPA-PEAP TKIP | 10 | Pass |
| 2 | MSI K7N420 Pro | Windows ME | Cisco 1200 | 802.1X LEAP | 5 | Pass |
| 3 | MSI-K8T Neo 2 | Windows XP x64 | Cisco 1200 | WPA-PSK + TKIP | 5 | Pass |
| 4 | ASUS P4T/WoA | Windows SP2 | DWL-8200 | WPA-PSK + TKIP | 11 | Pass |
| 5 | MSI K7N420 Pro | Windows ME | DWL-8200 | Open System + 128-bit ASCII Key | 1 | Pass |
| 6 | ASUS P4T/WOA | Windows 98SE | DWL-3200 | WPA2-PEAP AES | 11 | Pass |
| 7 | ASUS P5B-VM | Windows Vista x86 | DWL-8200 | WPA2-PSK + AES | 6 | Pass |
| 8 | ASUS P5B-VM | Windows Vista x86 | DI-784 | WPA2-PSK + AES | 6 | Pass |

### 5-2-2 Infrastructure mode

| Item | PC/Note Book | OS | AP | Encryption | Channel | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | ASUS P4T/WOA | Windows XP x64 | DI-624 C3 | WPA-PEAP TKIP | 6 | Pass |
| 2 | MSI-K8T Neo 2 | Windows XP x64 | DI-624 C3 | WPA-PSK + AES | 11 | Pass |
| 3 | ASUS P5B-VM | Win Vista x86 | DWL-8200 | WPA2-PSK + AES | 6 | Pass |
| 4 | ASUS P5B-VM | Win Vista x86 | DI-784 | WPA2-PSK + AES | 6 | Pass |

### 5-2-3 Ad-hoc mode

| Item | PC/Note Book | OS | Test Card | Encryption | Channel | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | ASUS P5B-VM | Win Vista x86 | DWL-630 | Open System + 128-bit Hex Key | 6 | Pass |
|  | ASUS P5P800 | Win Vista x86 | DWA-510 |  |  |  |

# 6. Other Related Test

## 6-1 WIFI Pretest

Test Environment

OS: Windows XP SP2 (WPA2)

| Item | Result (Pass/Fail/Not available) |
| --- | --- |
| WiFi | Pass |

## 6-2 Firewall

Test Environment

Tools: Ping command, Srv-U

File Size: 10MB

| OS | Connection Test | Firewall Disable | Firewall Enable |
| --- | --- | --- | --- |
| Windows XP SP2 (Server) | Ping Test | Pass | Pass |
|  | FTP Test | Pass | Pass |
|  | My Networks Places | Pass | Pass |
| Windows XP x64 SP1 (Server) | Ping Test | Pass | Pass |
|  | FTP Test | Pass | Pass |
|  | My Networks Places | Pass | Pass |

## 6-3 Multi-Language User Interface Test

Test Environment

Station: ASUS P5P800-VM [OS: Windows XP SP2]

Test Method: Check whether the Install-Shield, UI and Connection Wizard correspond to current language version of Windows XP.

| Item | Country | Result (Pass/Failed) |
| --- | --- | --- |
| 1 | English | Pass |
| 2 | Czech | Pass |
| 3 | Danish | Pass |
| 4 | Dutch | Pass |

| 5 | Finish | Pass |
| --- | --- | --- |
| 6 | French | Pass |
| 7 | German | Pass |
| 8 | Hungarian | Pass |
| 9 | Italian | Pass |
| 10 | Norwegian | Pass |
| 11 | Polish | Pass |
| 12 | Portuguese | Pass |
| 13 | Spanish | Pass |
| 14 | Swedish | Pass |
| 15 | Greek | Pass |
| 16 | Traditional Chinese | Pass |
| 17 | Simplified Chinese | Pass |
| 18 | Russian | Pass |

## 6-4 WMM Function Test

Test Environment

Test Tools: Airopeek

Test Software: Chariot 5.1

AP (Support WMM): Cisco 1200 AP

Server: ASUS (PUNDIT) AB-2800 R4R8L-2.8G

Station (Test): TOSHIBA A3 (Windows XP SP2)

Station (Test): ASUS P5P800-VM (Windows XP X64)

- The WMM function does not work in Windows Vista x86, refer to bug #1.

## 6-4-1 Windows XP SP2

##### 6-4-1-a Management Packet capturing

Use Wireless Airopeek tool capable of capturing and decoding WMM information (Association Request) is required

| OS | Result (Pass/Fail) |
| --- | --- |
| Windows XP SP2 | Pass |

##### 6-4-1-b Chariot Traffic characteristics

VI: Video

VO: Voice

BE: Best Effort

BK: Background

Test Script: The Chariot scripts which are to be used in the testing include the following:

Test Execution 1:

| Group | 802.11a Equipment | 802.11b Equipment | 802.11g Equipment |
| --- | --- | --- | --- |
| RTP1_BE | IPVT14Mbps–dn | IPVT3.5Mbps–dn | IPVT14Mbps–dn |
| RTP2_VI | IPTV10Mbps–dn | IPTV2.8Mbps–dn | IPTV10Mbps–dn |
| RTP3_BE | IPTV10MbpsDelay10sec-up | IPTV2.8MbpsDelay10sec-up | IPTV10MbpsDelay10sec-up |
| Result | N/A | 6.688 | 25.844 |

Test Execution 2:

| Group | 802.11a Equipment | 802.11b Equipment | 802.11g Equipment |
| --- | --- | --- | --- |
| RTP1_BE | IPVT14Mbps–up | IPVT3.5Mbps–up | IPVT14Mbps–up |
| RTP2_VI | IPTV10Mbps–up | IPTV2.8Mbps–up | IPTV10Mbps–up |
| RTP3_BE | IPTV10MbpsDelay10sec-dn | IPTV2.8MbpsDelay10sec-dn | IPTV10MbpsDelay10sec-dn |
| Result | N/A | 6.408 | 25.269 |

Test Execution 3:

| Group | 802.11a Equipment | 802.11b Equipment | 802.11g Equipment |
| --- | --- | --- | --- |
| RTP1_BE | IPVT10Mbps–dn | IPVT3.5Mbps–dn | IPVT10Mbps–dn |
| RTP2_VI | IPTV14Mbps–dn | IPTV2.8Mbps–dn | IPTV14Mbps–dn |
| RTP3_BE | IPTV14MbpsDelay10sec-dn | IPTV2.8MbpsDelay10sec-dn | IPTV14MbpsDelay10sec-dn |
| Result | N/A | 6.834 | 23.959 |

Test Execution 4:

| Group | 802.11a Equipment | 802.11b Equipment | 802.11g Equipment |
| --- | --- | --- | --- |
| RTP1_VI | IPVT10Mbps–up | IPVT3.5Mbps–up | IPVT10Mbps–up |
| RTP2_VO | IPTV14Mbps–up | IPTV2.8Mbps–up | IPTV14Mbps–up |
| RTP3_VI | IPTV14MbpsDelay10sec-dn | IPTV2.8MbpsDelay10sec-dn | IPTV14MbpsDelay10sec-dn |
| Result | N/A | 6.188 | 24.755 |

Test Execution 4:

| Group | 802.11a Equipment | 802.11b Equipment | 802.11g Equipment |
| --- | --- | --- | --- |
| RTP1_BK | IPVT10Mbps–up | IPVT3.5Mbps–up | IPVT10Mbps–up |
| RTP2_BE | IPTV14Mbps–up | IPTV2.8Mbps–up | IPTV14Mbps–up |
| RTP3_BK | IPTV14MbpsDelay10sec-dn | IPTV2.8MbpsDelay10sec-dn | IPTV14MbpsDelay10sec-dn |
| Result | N/A | 6.345 | 25.350 |

## 6-4-2 Windows XP X64

##### 6-4-2-a  Management Packet capturing

Use Wireless Airopeek tool capable of capturing and decoding WMM information (Association Request) is required

| OS | Result (Pass/Fail) | Remark |
| --- | --- | --- |
| Win XP_x64 | Pass |  |

##### 6-4-2-b  Chariot Traffic characteristics

VI: Video

VO: Voice

BE: Best Effort

BK: Background

Test Script: The Chariot scripts which are to be used in the testing include the following:

Test Execution 1:

| Group | 802.11a Equipment | 802.11b Equipment | 802.11g Equipment |
| --- | --- | --- | --- |
| RTP1_BE | IPVT14Mbps–dn | IPVT3.5Mbps–dn | IPVT14Mbps–dn |
| RTP2_VI | IPTV10Mbps–dn | IPTV2.8Mbps–dn | IPTV10Mbps–dn |
| RTP3_BE | IPTV10MbpsDelay10sec-up | IPTV2.8MbpsDelay10sec-up | IPTV10MbpsDelay10sec-up |
| Result | N/A | 6.408 | 24.646 |

Test Execution 2:

| Group | 802.11a Equipment | 802.11b Equipment | 802.11g Equipment |
| --- | --- | --- | --- |
| RTP1_BE | IPVT14Mbps–up | IPVT3.5Mbps–up | IPVT14Mbps–up |
| RTP2_VI | IPTV10Mbps–up | IPTV2.8Mbps–up | IPTV10Mbps–up |
| RTP3_BE | IPTV10MbpsDelay10sec-up | IPTV2.8MbpsDelay10sec-up | IPTV10MbpsDelay10sec-up |
| Result | N/A | 6.443 | 24.506 |

Test Execution 3:

| Group | 802.11a Equipment | 802.11b Equipment | 802.11g Equipment |
| --- | --- | --- | --- |
| RTP1_BE | IPVT14Mbps–dn | IPVT3.5Mbps–dn | IPVT14Mbps–dn |
| RTP2_VI | IPTV11Mbps–dn | IPTV2.8Mbps–dn | IPTV11Mbps–dn |
| RTP3_BE | IPTV11MbpsDelay10sec-dn | IPTV2.8MbpsDelay10sec-dn | IPTV11MbpsDelay10sec-dn |
| Result | N/A | 6.832 | 25.673 |

Test Execution 4:

| Group | 802.11a Equipment | 802.11b Equipment | 802.11g Equipment |
| --- | --- | --- | --- |
| RTP1_BK | IPVT14Mbps–up | IPVT3.5Mbps–up | IPVT14Mbps–up |
| RTP2_BE | IPTV10Mbps–up | IPTV2.8Mbps–up | IPTV10Mbps–up |
| RTP3_BK | IPTV10MbpsDelay10sec-dn | IPTV2.8MbpsDelay10sec-dn | IPTV10MbpsDelay10sec-dn |
| Result | N/A | 6.200 | 24.879 |

Test Execution 5:

| Group | 802.11a Equipment | 802.11b Equipment | 802.11g Equipment |
| --- | --- | --- | --- |
| RTP1_VI | IPVT10Mbps–up | IPVT3.5Mbps–up | IPVT10Mbps–up |
| RTP2_VO | IPTV14Mbps–up | IPTV2.8Mbps–up | IPTV14Mbps–up |
| RTP3_VI | IPTV14MbpsDelay10sec-dn | IPTV2.8MbpsDelay10sec-dn | IPTV14MbpsDelay10sec-dn |
| Result | N/A | 6.321 | 24.522 |

## 6-5 WMM Power Save Function Test

Test Environment

Test Tools: Airopeek

Test Software: Chariot 5.1

AP (Support WMM Power Save): Broadcom BCM 94704AGR

Server: ASUS (PUNDIT) AB-2800 R4R8L-2.8G

Station (Test): ASUS P5P800-VM   (Windows XP SP2)

Test Procedure: Refer to WI-FI WMM Power Save test plan.

| Procedure | Result (Pass/Fail) |
| --- | --- |
| WMM Power Save Function | Pass |

## 6-6 Microsoft Certify Test

| OS | Driver Ver. | Submission ID | Driver with CAT Result (Pass/Fail) | Note |
| --- | --- | --- | --- | --- |
| Windows 2000 / XP/XPx64 | 1.1.1.0 | 1206401 | Pass |  |
| Windows Vista x86 | 2.0.3.0 | 1225829 | Pass |  |