DWL-G700AP

Date: May 18,2005

| Product name: | 802.11g/2.4GHz 54Mbps Wireless Access Piont |  |
| --- | --- | --- |
| Revision: | B1 |  |
| Available: | E/May, 2005 |  |
| Picture: |  |  |
| Why Sell This Product: | As margins for the basic 802.11b gear is slipping, we are phasing in more 802.11g products for you to boost wireless LAN profits. This product belongs to the cost-down 802.11g AirPlusG product line, and can supplement products like DWL-G510 and DWL-G630 wireless adapters. Selling Points - No 108Mbps Turbo mode, but lower cost, lower price. - 54Mbps speed. - More WPA and WPA2 functions added (WPA EAP, WPA PSK) |  |
| Key Features: | - 802.11g 2.4GHz standard - Up to 54Mbps wireless data transfer rates - 10/100BASE-TX port for connection to Ethernet or DSL/cable modem - Compatible with existing 802.11b wireless devices - Built-in DHCP server for multiple user access - 64/128-bit WEP data encryption - WPA and the latest standard WPA2 security support - Web-based configuration & management |  |
| Technical Details: | Network Standards - 802.11g wireless LAN - 802.11b wireless LAN - 802.3/802.3u 10BASE-T/100BASE-TX Ethernet - ANSI/IEEE 802.3 NWay auto-negotiation Media Access Control Protocol CSMA/CA with ACK Network Architecture Type Infra-structure mode Operating Frequency 2.4 to 2.4835 GHz ISM band Operating Channels - FCC: 11 - ETSI: 13 Radio and Modulation Schemes DQPSK, DBPSK, CCK, OFDM Transmit Output Power (for 802.11b) Minimum +16dBm typically Transmit Output Power (for 802.11g) Minimum +13dBm typically Security - 64/128-bit WEP data encryption - WPA EAP, WPA PSK - WPA2 AES - MAC address filtering - SSID broadcast disable function Antenna 2dBi Gain detachable external omni-directional dipole antenna with RP-SMA connector Device Management Web-based management through Internet Explorer v.6 or later, Netscape Navigator v.6 or later or other Java-enabled browser Operating Voltage 7.5VDC 1A Power Input Through external power adapter Certification - FCC Part 15.247 - ETS 300 328 |  |
| EMI/EMC Report | FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others |  |
| Safety |  TUV No.  UL No.  CSA No.  Others |  |
| Packaging Style |  Clamshell  Brown Box  Gift Box  Bulk  Others |  |
| Packaging Contents: | - DWL-G700AP Device - Detachable Antenna - CAT5 Ethernet cable - Power Adapter (7.5V/1A) - CD with Manual - QIG - Mounting Kits - Vertical Stand |  |
| UPC Code: | Part Number; UPC Code. EWLG700BEUB1; 790069; 281679. EWLG700EEUB1; 790069; 281686. IWLG700ANAB1; 790069; 281693. IWLG700BEUB1; 790069; 281709. IWLG700EEUB1; 790069; 281716. IWLG700ENAB1; 790069; 281723. IWLG700EISB1; 790069; 281730. IWLG700NEUB1; 790069; 281747. TWLG700ANAB1; 790069; 281822. NWL2K+ACEUB1; 790069; 281754. |
| Units Per Carton: | 10 |  |
| Cartons Per Pallet: | For WW by Air: 33 Sea: 44 | For EU by Air: 44 Sea: 44 |
| Dimensions and Weight: | Box: , Others mm, 1 kg Carton: 366*287*442 mm, 11 kg Pallet: (by Air) 120*110*150 cm, 378 kg (for WW) (by Sea) 120*110*195 cm, 499 kg (for WW) (by Air) 120*110*195 cm, 499 kg (for EU) (by Sea) 120*110*195 cm, 499 kg (for EU) INTRASTAT weight: 900 g (Box) Packing material weight (Box): - Paper: 80.5 g - Plastic: 15.5g - Mixed: 4 g |  |