DWL-G810

Date: April 22, 2004

| Product name: | Enhanced 2.4 GHz Ethernet to Wireless Bridge |
| --- | --- |
| Product Group & Type: |  |
| Revision: | A1 |
| Available: |  |
| Picture: |  |
| Why Sell This Product: | - Additional choice for game players & printer/Ethernet device wireless connection: With this new 802.11g DWL-G810 and and the current 802.11b+ DWL-810+, now you have 2 clear-cut products with 2 different speeds to offer to your customers. - Expand your wireless 802.11g AirPlusXtremeG product range. Selling Points - 108Mbps Super G high speed. - Award-winning product ("Pick-O-the-Crop" March 2004 award). |
| Keyfeatures: | - Easy, fast wireless connection for Ethernet devices - 802.11g wireless LAN standard - 2.4GHz frequency band - Up to 108Mbps Turbo mode data transfer rate - 10/100Mbps Ethernet port with auto MDI/MDIX connection to network device - WPA security, 64/128/152-bit WEP data encryption - Ad-hoc and infrastructure modes - Web-based configuration |
| Technical Details: | Standards - IEEE 802.11g wireless LAN - IEEE 802.3/802.3u 10BASE-T/100BASE-TX Fast Ethernet Adapter Type Fast Ethernet to 802.11g Wireless LAN Device Port 10/100BASE-TX Ethernet port with auto MDI/MDIX Media Access Control Protocol CSMA/CA with ACK Network Architecture Types Supports ad-hoc and infrastructure modes Operating Mode Wireless client Network Transfer Rate/Modulation Technique - 802.11b: 11Mbps, 5.5Mbps: CCK 2Mbps: DQPSK 1Mbps: DBPSK - 802.11g: 108Mbps Super G, 54Mbps, 48Mbps, 36Mbps, 24Mbps, 18Mbps, 12Mbps, 9Mbps, 6Mbps: OFDM (Orthogonal Frequency Division Multiplexing) Frequency Range 2.4 to 2.4835 GHz Operating Channels - 1 to 11 channels (FCC) - 1 to 13 channels (ETSI) Antenna 2dBi Gain detachable external dipole antenna (reverse SMA connector) Operating Range * - Indoor: up to 100 meters (328 feet) - Outdoor: up to 400 meters (1,312 feet) * Environmental factors may adversely affect range Transmitter Output Power 15dBm (32mW) +/- 2dB Data Security - WEP 64/128/152-bit data encryption (user-selectable) - Wi-Fi TM Protected Access with TKIP, MIC, IV Expansion, Shared Key Authentication Configuration/Management Web-based (Internet Explorer v.6 or later, Netscape Navigator v.7 or later, or other Java-enabled browsers) Physical & Environmental Diagnostic LEDs - Power - WLAN DC Input 5V 2.0A Power Supply Through external power adapter Dimensions 90 (L) x 82 (W) x 40 (H) mm Weight 153 grams Operating Temperature 0 to 55 C Storage Temperature -20 to 75 C Humidity Up to 95% non-condensing Emission Certification - FCC Class B - CE Class B Safety Certification UL |
| EMI/EMC Report | FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No. ACN052202838  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others |
| Safety |  TUV No.  UL No.  CSA No.  Others |
| Packaging Style |  Clamshell  White Box  Gift Box  Bulk  Others |
| Packaging Contents: | DWL-G810 Ethernet Wireless Bridge MasterCD(Manual,QIG)QIG, Ethernet Cable, AC power Adapter |
| UPC Code: | BDWLG810..B1; 790069262043. CDWLG810..B1; 790069262043. DWLG810ANAB1; 790069262647. DWLG810ENAB1; 790069262654. DWLG810BEUB1; 790069262661. DWLG810EEUB1; 790069262678. DWLG810NEUB1; 790069262685. DWLG810CN.B1; 790069262692. TDWLG810..B1; 790069262715. |
| Units Per Carton: | 5 |
| Cartons Per Pallet: | Air: 60, Sea: 90 |
| Dimensions and Weight: | Box: 275 x 208 x 66 mm, 0.81 kg Carton: 440 x 360 x 290 mm, 5.15 kg Pallet: (by Air) 1200 x 1100 x 1600 mm, 260 kg (by Sea) 1200 x 1100 x 2000 mm, 380 kg INTRASTAT weight: kg (Box) Packing material weight (Box): - Paper: kg - Plastic: kg - Mixed: kg |