Please translate …..THANKs

Warning: Please follow the instructions in this guide carefully.  Connecting these adapters incorrectly may result in damage to you network devices.

DWL-P50

Power Over Ethernet Adapters

About DWL-P50

The DWL-P50 Power Over Ethernet (POE) Adapters are designed to work with D-Link PoE switches to deliver power over Ethernet to D-Link wireless Access Point, IP Camera and IP phone, which didn’t support PoE. This allows you to place those D-Link devices virtually anywhere in a building where power outlets are not available. DWL-P50 Unit splits the data and DC power apart form D-Link PoE switch and then supplies them to D-Link wireless Access Point, IP Camera and IP phone. (Please refer appendix A for DWL-P50 compatibility with D-Link products )

## Check Your Package Contents

- DWL-P50 Power Over Ethernet Adapter

- Ethernet (CAT5 UTP/Straight Through) Cable

- Female-to-Female Power Connector

Connecting The DWL-P50

DC OUT: supplies 5V  or 12V of power. Using the included power cable, connect to your Ethernet enabled device’s power.

Voltage Selector: used to select DC 5V (11.5W) or 12V (12W)

LAN OUT: connect a CAT 5 RJ-45 straight- through cable to an Ethernet enabled device.

P+DATA IN: connect a CAT 5 RJ-45 straight- through cable from PoE switch.

Use the Voltage Selector to set the DC output to 5V(11.5W) or 12V(12W) depending upon your Network device’s power specifications.

Connect a Cat 5 Ethernet cable from the PoE port of switch (For example: 1~8 port of D-Link DES-1316) to the “P+DATA IN” of DWL-P50

Use a CAT 5, 5e, or 6 UTP/STP straight-through Ethernet cable.

Insert the Female-to-Female Power cable into “DC OUT” and the power receptor of the Network Device. The Power LED of the Network Device should light up.

Warning: Select voltage that your device requires before connect power cable to power

receptor. Incorrect voltage selection may damage product.

Connect the provided Cat 5 Ethernet cable from DWL-P50 “LAN OUT” to the RJ-45 LAN port of the Network Device.

Installation is complete! Your Network Device should have power and data, and function normally.

## Features

- IEEE 802.3af compatible

- Optional output 5VDC or 12VDC

- No Software Required and Operating System Independent

- Work with D-Link PoE switches DES-1316 to provide Wireless AP, Internet Camera, IP Phone or Equivalent 5VDC or 12VDC System

## Specifications

Voltage

. Output: 5VDC or 12VDC

Power Output

. Output: 11.5W(5VDC) or 12W(12VDC)

Ethernet

. Connector: RJ-45

. Data Rate: 10/100Mbps

. Cable: TIA/EIA - 568, Category 5

Dimensions

8.09 x 5.2 x 2.8cm

# Weight

80grams

Temperature

- Operating: 0℃ to 40℃ (32°F to 104°F)

- Storage: -40℃ to 70℃ (-40°F to 158°F)

Humidity

-	Operation: 10%-80% RH

-	Storage: 5% ~ 90% RH